Integrated Effect-Based Analysis of Endocrine Disruption Risks in Advanced Wastewater Treatment: Elevated Hazards from Ozonation and UV Processes

Yingnan Han 1, Kongrui Zhu 2, Na Li 2, Wei Shang 3, Yu Zhang 1,4, Mei Ma 1, 4 *

1 Key Laboratory of Environmental Aquatic Chemistry, State Key Laboratory of Regional Environment and Sustainability, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing 100085, P. R. China

2 School of Biomedical Engineering, Tsinghua University, Beijing, China

3 North China Municipal Engineering Design & Research Institute Co., LTD

4 University of Chinese Academy of Sciences, Beijing 100049, China

Table S1 BEQs of different nuclear receptor disrupting effects for each sample

| Treatment Processes | BEQ (E2, ng/L) | BEQ (4-OHT, μg/L) | BEQ (FLU, μg/L) | BEQ (AMD, mg/L) |
| --- | --- | --- | --- | --- |
| Influent | 0.59±0.041 | 58±2.1 | 156±9.4 | 0.51±0.020 |
| Preliminary Sedimentation | 0.23±0.023 | 37±2.5 | 51±4.5 | 0.43±0.031 |
| A²O | 0.15±0.020 | 15±1.1 | 12±4.7 | 0.21±0.016 |
| Secondary Sedimentation | — | 11±1.6 | 3.8±2.6 | 0.21±0.016 |
| UV | — | 9.2±0.21 | 23±5.7 | 0.088±0.0085 |
| P1 Influent | — | 22±3.1 | 4.8±0.91 | 0.26±0.0060 |
| P1 Ozonation | — | 7.8±0.13 | 0.63±0.093 | 0.10±0.0059 |
| P2 Influent | 0.010±0.023 | 22±1.9 | 9.96±1.42 | 0.20±0.012 |
| P2 Coagulation | — | 20±1.9 | 5.0±0.59 | 0.33±0.038 |
| P2 Ozonation | — | 7.6±0.32 | 16±3.1 | 0.11±0.0068 |
| P2 BAC | 0.093±0.0057 | 8.1±0.074 | 16±2.2 | 0.10±0.0027 |

"—" indicates a negative result for this sample.

Fig. S1. Agonist activity to ER of positive chemical E2

Fig. S2 Anti-agonist activity to ER of positive chemical 4-OHT

Fig. S3 Agonist activity to AR of positive chemical DHT

Fig. S4 Anti-agonist activity to AR of positive chemical FLU

Fig. S5 Agonist activity to TR of positive chemical T3

Fig. S6 Anti-agonist activity to TR of positive chemical AMD

Fig. S7 Agonist activity to ER of different concentration ratios of WWTP water samples

Fig. S8 Anti-agonist activity to ER of different concentration ratios of WWTP water sample

Fig. S9 Anti-agonist activity to AR of different concentration ratios of WWTP water samples

Fig. S10 Anti-agonist activity to TR of different concentration ratios of WWTP water samples

Appendix S1: EBT value derivation for the anti-AR activity by two-hybrid yeast assay

In brief, 11 anti-AR positive compounds were tested using recombinant androgen nuclear receptor gene two-hybrid yeast assay, and relative effect potencies (REP) to Flutamide were calculated, as shown in Table S2.

Ecotoxicity data for these positive compounds with relative effect potencies (REP) values > 0.001 were collected from multiple databases and converted into BEQ of Flutamide based on their REP. The final BEQ was adjusted by a factor according to the measurement endpoint. The ecotoxicity data for the positive componds and their BEQ data were listed in Table S3. The lowest BEQ value was selected as the safe-BEQ (0.511 ng FLU EQ/L). The HC5-BEQ for the reference compound was estimated using species sensitivity distribution (SSD) analysis. SSD curves were generated from EC50-BEQ values using the statistical software ETX 2.3. The HC5-BEQ was calculated to be 7.97 μg Flu EQ/L (Fig. S11).

Further, water samples collected from drinking water sources in six Chinese cities were determined to obtain the clean-BEQ (Table S4). The clean BEQ was the average value of six cities as 0.916 ± 0.587 μg Flu EQ/L.

The EBT was calculated by integrating the HC5-BEQ, clean-BEQ, and safe-BEQ according to assessment factors and multiplication factors (van der Oost et al., 2017). The criterion for calculating EBT is based on the following four principles.

(1) If clean-BEQ < HC5-BEQ, EBT ≈ HC5-BEQ;

(2) If clean-BEQ ≪ HC5-BEQ, EBT = 2-10 × safe-BEQ;

(3) If clean-BEQ ≈ HC5-BEQ, EBT within HC5-BEQ 95 % confidence interval;

(4) If clean-BEQ > HC5-BEQ, EBT ≈ 2-5 × clean-BEQ.

Finally, the effect trigger value for the recombinant AR gene two-hybrid yeast assay was derivated to be 7.97 μg FLU EQ/L.

Table S2 REP of 11 anti-AR activity positive compound

| chemicals | REP | CAS |
| --- | --- | --- |
| (R)-Bicalutamide | 0.45 | 90357-06-5 |
| 4-n-Octylphenol | 158.40 | 1806-26-4 |
| diisononylphthalate | 4.16 | 28553-12-0 |
| finasteride | 0.024 | 98319-26-7 |
| 17a-Ethinyl estradiol | 1.42 | 57-63-6 |
| 4-hydroxyazobenzene | 6.52 | 1689-82-3 |
| Atrazine | 1.12 | 1912-24-9 |
| Benzo [a] pyrene | 0.32 | 50-32-8 |
| Captan | 0.92 | 133-06-2 |
| Diethylstilbestrol | 6.40 | 56-53-1 |
| Flutamide | 1.00 | 13311-84-7 |

Table S3 Ecotoxicity data for the positive compounds

| CAS Number | Species Scientific Name | Species Common Name | Species Group | EC50 (μg/L) | BEQ (μg/L) | Effect | Endpoint |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 57636 | Gobiocypris rarus | Chinese Rare Minnow | Fish | 0.00036 | 0.000511088 | Mortality | LOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.00152 | 0.002157929 | Mortality | LOEC |
| 57636 | Oncorhynchus mykiss | Rainbow Trout | Fish | 0.0016 | 0.002271504 | Mortality | LOEC |
| 57636 | Gobiocypris rarus | Chinese Rare Minnow | Fish | 0.0018 | 0.002555442 | Mortality | NOEC |
| 57636 | Gobiocypris rarus | Chinese Rare Minnow | Fish | 0.00182 | 0.002583836 | Mortality | LOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.0025 | 0.003549225 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.004 | 0.00567876 | Mortality | LOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.004 | 0.00567876 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.005 | 0.00709845 | Mortality | NOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.00644 | 0.009142804 | Mortality | LOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.00644 | 0.009142804 | Mortality | LOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.0076 | 0.010789644 | Mortality | NOEC |
| 57636 | Gobiocypris rarus | Chinese Rare Minnow | Fish | 0.0091 | 0.012919179 | Mortality | NOEC |
| 57636 | Gobiocypris rarus | Chinese Rare Minnow | Fish | 0.0091 | 0.012919179 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.01 | 0.0141969 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.01 | 0.0141969 | Mortality | NOEC |
| 57636 | Fundulus heteroclitus | Mummichog | Fish | 0.01 | 0.0141969 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.01 | 0.0141969 | Mortality | LC50 |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.01 | 0.0141969 | Mortality | LC50 |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.011 | 0.01561659 | Mortality | NOEC |
| 57636 | Lithobates clamitans ssp. clamitans | Bronze Frog | Amphibians | 0.0122 | 0.017320218 | Mortality | LOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.0186 | 0.026406234 | Mortality | LOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.02 | 0.0283938 | Mortality | LOEC |
| 57636 | Fundulus heteroclitus | Mummichog | Fish | 0.02 | 0.0283938 | Mortality | LOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.02 | 0.0283938 | Mortality | LOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.02 | 0.0283938 | Mortality | LOEC |
| 50328 | Palaemon pugio | Daggerblade Grass Shrimp | Crustaceans | 0.102 | 0.033055242 | Mortality | LC50 |
| 57636 | Gobiocypris rarus | Chinese Rare Minnow | Fish | 0.02722 | 0.038643962 | Mortality | LOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.031 | 0.04401039 | Mortality | LOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.031 | 0.04401039 | Mortality | LOEC |
| 57636 | Gobiocypris rarus | Chinese Rare Minnow | Fish | 0.0361 | 0.051250809 | Mortality | NOEC |
| 57636 | Rutilus rutilus | Roach | Fish | 0.04 | 0.0567876 | Mortality | NOEC |
| 57636 | Rutilus rutilus | Roach | Fish | 0.04 | 0.0567876 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.045 | 0.06388605 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.045 | 0.06388605 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.045 | 0.06388605 | Mortality | NOEC |
| 1912249 | Deleatidium sp. | Mayfly | Insects | 0.06 | 0.06709446 | Mortality | LC50 |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.0495 | 0.070274655 | Mortality | NOEC |
| 57636 | Lithobates septentrionalis | Mink Frog | Amphibians | 0.05 | 0.0709845 | Mortality | NOEC |
| 57636 | Sparus aurata | Gilthead Seabream | Fish | 0.05 | 0.0709845 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.0558 | 0.079218702 | Mortality | NOEC |
| 57636 | Lithobates septentrionalis | Mink Frog | Amphibians | 0.061 | 0.08660109 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.078 | 0.11073582 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.078 | 0.11073582 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.078 | 0.11073582 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.093 | 0.13203117 | Mortality | NOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.0936 | 0.132882984 | Mortality | LOEC |
| 57636 | Sparus aurata | Gilthead Seabream | Fish | 0.1 | 0.141969 | Mortality | LOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.1 | 0.141969 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.1 | 0.141969 | Mortality | NOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.1 | 0.141969 | Mortality | NOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.1 | 0.141969 | Mortality | NOEC |
| 57636 | Fundulus heteroclitus | Mummichog | Fish | 0.1 | 0.141969 | Mortality | NOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.1 | 0.141969 | Mortality | NOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.1085 | 0.154036365 | Mortality | NOEC |
| 50328 | Daphnia pulex | Water Flea | Crustaceans | 0.5 | 0.1620355 | Mortality | LC50 |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.12 | 0.1703628 | Mortality | NOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.155 | 0.22005195 | Mortality | NOEC |
| 57636 | Fundulus heteroclitus | Mummichog | Fish | 0.2 | 0.283938 | Mortality | LOEC |
| 50328 | Daphnia magna | Water Flea | Crustaceans | 0.981506517 | 0.318077798 | Mortality | EC50 |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.25 | 0.3549225 | Mortality | NOEC |
| 57636 | Syngnathus abaster | Shortsnouted Pipefish | Fish | 0.2655 | 0.376927695 | Mortality | NOEC |
| 57636 | Rana temporaria | Frog | Amphibians | 0.26676846 | 0.378728515 | Mortality | NOEC |
| 133062 | Spirostomum ambiguum | Protozoa | Invertebrates | 0.5 | 0.4581455 | Mortality | LC50 |
| 50328 | Daphnia magna | Water Flea | Crustaceans | 1.624910532 | 0.526586381 | Mortality | EC50 |
| 50328 | Danio rerio | Zebra Danio | Fish | 1.7 | 0.5509207 | Mortality | NOEC |
| 50328 | Danio rerio | Zebra Danio | Fish | 1.7 | 0.5509207 | Mortality | NOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.468 | 0.66441492 | Mortality | NOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.468 | 0.66441492 | Mortality | NOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.468 | 0.66441492 | Mortality | NOEC |
| 57636 | Pimephales promelas | Fathead Minnow | Fish | 0.5 | 0.709845 | Mortality | NOEC |
| 57636 | Gobiocypris rarus | Chinese Rare Minnow | Fish | 0.5583 | 0.792612927 | Mortality | NOEC |
| 57636 | Gobiocypris rarus | Chinese Rare Minnow | Fish | 0.5583 | 0.792612927 | Mortality | NOEC |
| 57636 | Danio rerio | Zebra Danio | Fish | 0.5928188 | 0.841618922 | Mortality | LOEC |
| 57636 | Oncorhynchus mykiss | Rainbow Trout | Fish | 0.65 | 0.9227985 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 1 | 1.118241 | Mortality | NOEC |
| 57636 | Fundulus heteroclitus | Mummichog | Fish | 1 | 1.41969 | Mortality | NOEC |
| 57636 | Potamopyrgus antipodarum | Snail | Molluscs | 1 | 1.41969 | Mortality | NOEC |
| 50328 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 5.8 | 1.8796118 | Mortality | LC50 |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 2 | 2.236482 | Mortality | LOEC |
| 133062 | Salmo trutta | Brown Trout | Fish | 2.62 | 2.40068242 | Mortality | LC50 |
| 133062 | Salmo trutta | Brown Trout | Fish | 2.62 | 2.40068242 | Mortality | LC50 |
| 133062 | Salvelinus fontinalis | Brook Trout | Fish | 3.4 | 3.1153894 | Mortality | LC50 |
| 1912249 | Sigara arguta | Water Boatman | Insects | 2.904 | 3.247371864 | Mortality | LC50 |
| 133062 | Carassius auratus | Goldfish | Fish | 3.7 | 3.3902767 | Mortality | LC50 |
| 133062 | Carassius auratus | Goldfish | Fish | 3.7 | 3.3902767 | Mortality | LC50 |
| 133062 | Carassius auratus | Goldfish | Fish | 3.7 | 3.3902767 | Mortality | LC50 |
| 133062 | Spirostomum ambiguum | Protozoa | Invertebrates | 4 | 3.665164 | Mortality | EC50 |
| 133062 | Salvelinus namaycush | Lake Trout, Siscowet | Fish | 4.9 | 4.4898259 | Mortality | LC50 |
| 133062 | Salvelinus namaycush | Lake Trout, Siscowet | Fish | 5.1 | 4.6730841 | Mortality | LC50 |
| 133062 | Salvelinus namaycush | Lake Trout, Siscowet | Fish | 5.3 | 4.8563423 | Mortality | LC50 |
| 133062 | Oncorhynchus clarkii | Cutthroat Trout | Fish | 5.64 | 5.16788124 | Mortality | LC50 |
| 133062 | Oncorhynchus kisutch | Silver Salmon | Fish | 5.65 | 5.17704415 | Mortality | LC50 |
| 1912249 | Neomysis integer | Opossum Shrimp | Crustaceans | 4.8 | 5.3675568 | Mortality | LC50 |
| 133062 | Salvelinus namaycush | Lake Trout, Siscowet | Fish | 6.32 | 5.79095912 | Mortality | LC50 |
| 133062 | Salvelinus namaycush | Lake Trout, Siscowet | Fish | 6.32 | 5.79095912 | Mortality | LC50 |
| 133062 | Pimephales promelas | Fathead Minnow | Fish | 6.5 | 5.9558915 | Mortality | LC50 |
| 57636 | Oryzias latipes | Japanese Medaka | Fish | 4.24 | 6.0194856 | Mortality | NOEC |
| 133062 | Lepomis macrochirus | Bluegill | Fish | 7.2 | 6.5972952 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 7.32 | 6.70725012 | Mortality | LC50 |
| 1912249 | Lithobates pipiens | Leopard Frog | Amphibians | 6 | 6.709446 | Mortality | LOEC |
| 133062 | Oncorhynchus clarkii | Cutthroat Trout | Fish | 7.41 | 6.78971631 | Mortality | LC50 |
| 57636 | Oryzias latipes | Japanese Medaka | Fish | 4.8 | 6.814512 | Mortality | NOEC |
| 133062 | Salvelinus namaycush | Lake Trout, Siscowet | Fish | 7.5 | 6.8721825 | Mortality | LC50 |
| 133062 | Oncorhynchus kisutch | Silver Salmon | Fish | 7.5 | 6.8721825 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 7.5 | 6.8721825 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 7.5 | 6.8721825 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 7.5 | 6.8721825 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 7.5 | 6.8721825 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 7.5 | 6.8721825 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 7.64 | 7.00046324 | Mortality | LC50 |
| 133062 | Ictalurus punctatus | Channel Catfish | Fish | 7.75 | 7.10125525 | Mortality | LC50 |
| 133062 | Ictalurus punctatus | Channel Catfish | Fish | 7.98 | 7.31200218 | Mortality | LC50 |
| 133062 | Salmo trutta | Brown Trout | Fish | 8 | 7.330328 | Mortality | LC50 |
| 133062 | Salmo trutta | Brown Trout | Fish | 8.1 | 7.4219571 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 8.4 | 7.6968444 | Mortality | LC50 |
| 1912249 | Zostera marina | Eelgrass | Plants | 7 | 7.827687 | Mortality | LC50 |
| 50328 | Daphnia magna | Water Flea | Crustaceans | 25 | 8.101775 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 9 | 8.246619 | Mortality | LC50 |
| 1912249 | Ambystoma barbouri | Streamside Salamander | Amphibians | 8 | 8.945928 | Mortality | LOEC |
| 1912249 | Ambystoma barbouri | Streamside Salamander | Amphibians | 8 | 8.945928 | Mortality | LOEC |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 10 | 9.16291 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 10 | 9.16291 | Mortality | LC50 |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 9.173 | 10.25762469 | Mortality | LC50 |
| 1912249 | Alaria sp. | Trematode | Worms | 9.234 | 10.32583739 | Mortality | LC50 |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 9.4 | 10.5114654 | Mortality | LC50 |
| 133062 | Perca flavescens | Yellow Perch | Fish | 12 | 10.995492 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 12 | 10.995492 | Mortality | LC50 |
| 133062 | Oncorhynchus tshawytscha | Chinook Salmon | Fish | 12 | 10.995492 | Mortality | LC50 |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 10 | 11.18241 | Mortality | NOEC |
| 1912249 | Zostera marina | Eelgrass | Plants | 10 | 11.18241 | Mortality | LC50 |
| 1912249 | Zostera marina | Eelgrass | Plants | 10 | 11.18241 | Mortality | LC50 |
| 1912249 | Xanthocnemis zealandica | Damselfly | Insects | 10 | 11.18241 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 13 | 11.911783 | Mortality | LC50 |
| 133062 | Pimephales promelas | Fathead Minnow | Fish | 13.4 | 12.2782994 | Mortality | LC50 |
| 1912249 | Echinostoma trivolvis | Trematode | Worms | 11.009 | 12.31071517 | Mortality | LC50 |
| 133062 | Oncorhynchus kisutch | Silver Salmon | Fish | 13.7 | 12.5531867 | Mortality | LC50 |
| 133062 | Oncorhynchus kisutch | Silver Salmon | Fish | 13.7 | 12.5531867 | Mortality | LC50 |
| 133062 | Oncorhynchus tshawytscha | Chinook Salmon | Fish | 13.9 | 12.7364449 | Mortality | LC50 |
| 133062 | Lepomis macrochirus | Bluegill | Fish | 14.1 | 12.9197031 | Mortality | LC50 |
| 133062 | Lepomis macrochirus | Bluegill | Fish | 14.1 | 12.9197031 | Mortality | LC50 |
| 50328 | Physella acuta | European Physa | Molluscs | 40 | 12.96284 | Mortality | LOEC |
| 133062 | Lepomis macrochirus | Bluegill | Fish | 14.5 | 13.2862195 | Mortality | LC50 |
| 1912249 | Tigriopus brevicornis | Harpacticoid Copepod | Crustaceans | 12.09 | 13.51953369 | Mortality | LC50 |
| 1912249 | Tigriopus brevicornis | Harpacticoid Copepod | Crustaceans | 12.45 | 13.92210045 | Mortality | LC50 |
| 133062 | Pimephales promelas | Fathead Minnow | Fish | 15.2 | 13.9276232 | Mortality | LC50 |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 12.5 | 13.9780125 | Mortality | LC50 |
| 133062 | Perca flavescens | Yellow Perch | Fish | 15.4 | 14.1108814 | Mortality | LC50 |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 10 | 14.1969 | Mortality | LC50 |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 10 | 14.1969 | Mortality | LC50 |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 10 | 14.1969 | Mortality | LC50 |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 10 | 14.1969 | Mortality | LC50 |
| 57636 | Danio rerio | Zebra Danio | Fish | 10 | 14.1969 | Mortality | LOEC |
| 57636 | Fundulus heteroclitus | Mummichog | Fish | 10 | 14.1969 | Mortality | NOEC |
| 57636 | Physella pomilia | Claiborne Physa | Molluscs | 10 | 14.1969 | Mortality | NOEC |
| 57636 | Physella pomilia | Claiborne Physa | Molluscs | 10 | 14.1969 | Mortality | NOEC |
| 133062 | Lepomis macrochirus | Bluegill | Fish | 16 | 14.660656 | Mortality | LC50 |
| 50328 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 46 | 14.907266 | Mortality | LOEC |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 18 | 16.493238 | Mortality | LC50 |
| 1912249 | Tigriopus brevicornis | Harpacticoid Copepod | Crustaceans | 15.32 | 17.13145212 | Mortality | LC50 |
| 133062 | Bufo bufo ssp. japonicus | Toad | Amphibians | 20 | 18.32582 | Mortality | LC50 |
| 133062 | Pimephales promelas | Fathead Minnow | Fish | 20 | 18.32582 | Mortality | LC50 |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 3.16 | 20.23854548 | Mortality | LC50 |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 3.16 | 20.23854548 | Mortality | LC50 |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 3.16 | 20.23854548 | Mortality | LC50 |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 3.16 | 20.23854548 | Mortality | LC50 |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 20 | 22.36482 | Mortality | LOEC |
| 1912249 | Arrenurus sp. | Water Mite | Insects | 20 | 22.36482 | Mortality | EC50 |
| 133062 | Acheilognathus moriokae | Minnow, Tanago | Fish | 25 | 22.907275 | Mortality | LC50 |
| 133062 | Cyprinus carpio | Common Carp | Fish | 25 | 22.907275 | Mortality | LC50 |
| 133062 | Cyprinus carpio | Common Carp | Fish | 25 | 22.907275 | Mortality | LC50 |
| 133062 | Cyprinus carpio | Common Carp | Fish | 25 | 22.907275 | Mortality | LC50 |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 21.01 | 23.49424341 | Mortality | LC50 |
| 1912249 | Ictalurus punctatus | Channel Catfish | Fish | 22 | 24.601302 | Mortality | LC50 |
| 1912249 | Ictalurus punctatus | Channel Catfish | Fish | 22 | 24.601302 | Mortality | LC50 |
| 50328 | Physella acuta | European Physa | Molluscs | 80 | 25.92568 | Mortality | LOEC |
| 50328 | Danio rerio | Zebra Danio | Fish | 81.39691578 | 26.37837989 | Mortality | LC50 |
| 133062 | Pimephales promelas | Fathead Minnow | Fish | 29 | 26.572439 | Mortality | LC50 |
| 1912249 | Diporeia sp. | Amphipod | Crustaceans | 24 | 26.837784 | Mortality | LC50 |
| 1912249 | Ictalurus punctatus | Channel Catfish | Fish | 24 | 26.837784 | Mortality | LC50 |
| 133062 | Rasbora heteromorpha | Harlequinfish, Red Rasbora | Fish | 30 | 27.48873 | Mortality | LC50 |
| 133062 | Anguilla japonica | Japanese Eel | Fish | 30 | 27.48873 | Mortality | LC50 |
| 57636 | Fundulus heteroclitus | Mummichog | Fish | 20 | 28.3938 | Mortality | LOEC |
| 57636 | Poecilia reticulata | Guppy | Fish | 20 | 28.3938 | Mortality | NOEC |
| 133062 | Lepomis macrochirus | Bluegill | Fish | 31 | 28.405021 | Mortality | LC50 |
| 133062 | Rasbora heteromorpha | Harlequinfish, Red Rasbora | Fish | 33 | 30.237603 | Mortality | LC50 |
| 133062 | Misgurnus anguillicaudatus | Oriental Weatherfish | Fish | 34 | 31.153894 | Mortality | LC50 |
| 50328 | Danio rerio | Zebra Danio | Fish | 100 | 32.4071 | Mortality | NOEC |
| 50328 | Danio rerio | Zebra Danio | Fish | 100 | 32.4071 | Mortality | NOEC |
| 50328 | Oryzias latipes | Japanese Medaka | Fish | 100 | 32.4071 | Mortality | NOEC |
| 50328 | Physella acuta | European Physa | Molluscs | 100 | 32.4071 | Mortality | NOEC |
| 50328 | Neanthes arenaceodentata | Polychaete Worm | Worms | 100 | 32.4071 | Mortality | LC50 |
| 50328 | Haliotis diversicolor | Abalone | Molluscs | 100.5 | 32.5691355 | Mortality | LC50 |
| 28553120 | Paratanytarsus parthenogeneticus | Midge | Insects | 8 | 33.279816 | Mortality | LC50 |
| 1912249 | Anaxyrus terrestris | Southern Toad | Amphibians | 30 | 33.54723 | Mortality | NOEC |
| 1912249 | Hyla versicolor | Gray Tree Frog | Amphibians | 30 | 33.54723 | Mortality | NOEC |
| 1912249 | Anaxyrus terrestris | Southern Toad | Amphibians | 30 | 33.54723 | Mortality | NOEC |
| 1912249 | Pseudacris triseriata | Striped, Northern Chorus Frog | Amphibians | 30 | 33.54723 | Mortality | NOEC |
| 1912249 | Lithobates pipiens | Leopard Frog | Amphibians | 30 | 33.54723 | Mortality | NOEC |
| 1912249 | Bufo americanus | American Toad | Amphibians | 30 | 33.54723 | Mortality | NOEC |
| 50328 | Danio rerio | Zebra Danio | Fish | 103.6258937 | 33.582147 | Mortality | LC50 |
| 1912249 | Ictalurus punctatus | Channel Catfish | Fish | 31 | 34.665471 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 38 | 34.819058 | Mortality | LC50 |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 31.54 | 35.26932114 | Mortality | LC50 |
| 28553120 | Daphnia magna | Water Flea | Crustaceans | 8.6 | 35.7758022 | Mortality | LC50 |
| 28553120 | Daphnia magna | Water Flea | Crustaceans | 8.6 | 35.7758022 | Mortality | LC50 |
| 1912249 | Paratya australiensis | Shrimp | Crustaceans | 34 | 38.020194 | Mortality | LC50 |
| 1912249 | Pseudaphritis urvillii | Blennie | Fish | 34 | 38.020194 | Mortality | LC50 |
| 1912249 | Galaxias maculatus | Common Jollytail | Fish | 34 | 38.020194 | Mortality | LC50 |
| 1912249 | Ictalurus punctatus | Channel Catfish | Fish | 34 | 38.020194 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 34 | 38.020194 | Mortality | LC50 |
| 50328 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 120 | 38.88852 | Mortality | NOEC |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 35.8 | 40.0330278 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 44 | 40.316804 | Mortality | LC50 |
| 1912249 | Zostera marina | Eelgrass | Plants | 36.5 | 40.8157965 | Mortality | LC50 |
| 1912249 | Zostera marina | Eelgrass | Plants | 36.5 | 40.8157965 | Mortality | LC50 |
| 1912249 | Zostera marina | Eelgrass | Plants | 36.7 | 41.0394447 | Mortality | LC50 |
| 1912249 | Zostera marina | Eelgrass | Plants | 36.7 | 41.0394447 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 45 | 41.233095 | Mortality | LC50 |
| 90357065 | Pimephales promelas | Fathead Minnow | Fish | 92.31 | 41.54356164 | Mortality | NOEC |
| 28553120 | Pimephales promelas | Fathead Minnow | Fish | 10 | 41.59977 | Mortality | LC50 |
| 50328 | Danio rerio | Zebra Danio | Fish | 128.680803 | 41.70171651 | Mortality | LC50 |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 37.39 | 41.81103099 | Mortality | LC50 |
| 133062 | Rasbora heteromorpha | Harlequinfish, Red Rasbora | Fish | 46 | 42.149386 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 49 | 44.898259 | Mortality | LC50 |
| 133062 | Oryzias latipes | Japanese Medaka | Fish | 50 | 45.81455 | Mortality | LC50 |
| 133062 | Oryzias latipes | Japanese Medaka | Fish | 50 | 45.81455 | Mortality | LC50 |
| 1912249 | Lithobates catesbeianus | American Bullfrog | Amphibians | 41 | 45.847881 | Mortality | LC50 |
| 28553120 | Paratanytarsus parthenogeneticus | Midge | Insects | 12 | 49.919724 | Mortality | LC50 |
| 28553120 | Paratanytarsus parthenogeneticus | Midge | Insects | 12 | 49.919724 | Mortality | LC50 |
| 133062 | Clarias batrachus | Walking Catfish | Fish | 54.73 | 50.14860643 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 57 | 52.228587 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 57 | 52.228587 | Mortality | LC50 |
| 133062 | Anguilla japonica | Japanese Eel | Fish | 58 | 53.144878 | Mortality | LC50 |
| 133062 | Oryzias latipes | Japanese Medaka | Fish | 61 | 55.893751 | Mortality | LC50 |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 50 | 55.91205 | Mortality | LC50 |
| 1912249 | Rasbora heteromorpha | Harlequinfish, Red Rasbora | Fish | 50 | 55.91205 | Mortality | LC50 |
| 28553120 | Pimephales promelas | Fathead Minnow | Fish | 14 | 58.239678 | Mortality | LC50 |
| 28553120 | Pimephales promelas | Fathead Minnow | Fish | 14 | 58.239678 | Mortality | LC50 |
| 28553120 | Pimephales promelas | Fathead Minnow | Fish | 14 | 58.239678 | Mortality | LC50 |
| 28553120 | Pimephales promelas | Fathead Minnow | Fish | 14 | 58.239678 | Mortality | LC50 |
| 28553120 | Lepomis macrochirus | Bluegill | Fish | 14 | 58.239678 | Mortality | LC50 |
| 1912249 | Zostera marina | Eelgrass | Plants | 54 | 60.385014 | Mortality | LC50 |
| 1912249 | Zostera marina | Eelgrass | Plants | 54 | 60.385014 | Mortality | LC50 |
| 133062 | Oryzias latipes | Japanese Medaka | Fish | 66 | 60.475206 | Mortality | LC50 |
| 133062 | Danio rerio | Zebra Danio | Fish | 67 | 61.391497 | Mortality | LC50 |
| 1912249 | Rasbora heteromorpha | Harlequinfish, Red Rasbora | Fish | 55 | 61.503255 | Mortality | LC50 |
| 133062 | Danio rerio | Zebra Danio | Fish | 70 | 64.14037 | Mortality | LC50 |
| 50328 | Physella acuta | European Physa | Molluscs | 200 | 64.8142 | Mortality | NOEC |
| 1912249 | Ceriodaphnia dubia | Water Flea | Crustaceans | 58 | 64.857978 | Mortality | NOEC |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 58 | 64.857978 | Mortality | NOEC |
| 28553120 | Oncorhynchus mykiss | Rainbow Trout | Fish | 16 | 66.559632 | Mortality | LC50 |
| 28553120 | Oncorhynchus mykiss | Rainbow Trout | Fish | 16 | 66.559632 | Mortality | LC50 |
| 28553120 | Oncorhynchus mykiss | Rainbow Trout | Fish | 16 | 66.559632 | Mortality | LC50 |
| 28553120 | Oncorhynchus mykiss | Rainbow Trout | Fish | 16 | 66.559632 | Mortality | LC50 |
| 28553120 | Oncorhynchus mykiss | Rainbow Trout | Fish | 16 | 66.559632 | Mortality | LC50 |
| 1912249 | Scenedesmus acutus var. acutus | Green Algae | Algae | 59.745 | 66.80930855 | Mortality | LC50 |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 60.3 | 67.4299323 | Mortality | LC50 |
| 28553120 | Lepomis macrochirus | Bluegill | Fish | 17 | 70.719609 | Mortality | LC50 |
| 28553120 | Lepomis macrochirus | Bluegill | Fish | 17 | 70.719609 | Mortality | LC50 |
| 28553120 | Lepomis macrochirus | Bluegill | Fish | 17 | 70.719609 | Mortality | LC50 |
| 28553120 | Lepomis macrochirus | Bluegill | Fish | 17 | 70.719609 | Mortality | LC50 |
| 1912249 | Hyla versicolor | Gray Tree Frog | Amphibians | 64 | 71.567424 | Mortality | NOEC |
| 1912249 | Lithobates pipiens | Leopard Frog | Amphibians | 64 | 71.567424 | Mortality | NOEC |
| 57636 | Nitocra spinipes | Harpacticoid Copepod | Crustaceans | 51 | 72.40419 | Mortality | LC50 |
| 133062 | Oryzias latipes | Japanese Medaka | Fish | 80 | 73.30328 | Mortality | LC50 |
| 50328 | Danio rerio | Zebra Danio | Fish | 240 | 77.77704 | Mortality | NOEC |
| 50328 | Danio rerio | Zebra Danio | Fish | 240 | 77.77704 | Mortality | NOEC |
| 28553120 | Pimephales promelas | Fathead Minnow | Fish | 19 | 79.039563 | Mortality | LC50 |
| 28553120 | Pimephales promelas | Fathead Minnow | Fish | 19 | 79.039563 | Mortality | LC50 |
| 28553120 | Pimephales promelas | Fathead Minnow | Fish | 19 | 79.039563 | Mortality | LC50 |
| 28553120 | Pimephales promelas | Fathead Minnow | Fish | 19 | 79.039563 | Mortality | LC50 |
| 57636 | Lithobates sylvaticus | Wood Frog | Amphibians | 56.0213766 | 79.53298815 | Mortality | LC50 |
| 1912249 | Scenedesmus acutus var. acutus | Green Algae | Algae | 71.26 | 79.68585366 | Mortality | LC50 |
| 1912249 | Chironomus tentans | Midge | Insects | 72 | 80.513352 | Mortality | LC50 |
| 133062 | Carassius auratus | Goldfish | Fish | 89 | 81.549899 | Mortality | LC50 |
| 133062 | Oryzias latipes | Japanese Medaka | Fish | 90 | 82.46619 | Mortality | LC50 |
| 90357065 | Pimephales promelas | Fathead Minnow | Fish | 184.222 | 82.90800577 | Mortality | LOEC |
| 133062 | Daphnia magna | Water Flea | Crustaceans | 92 | 84.298772 | Mortality | LC50 |
| 133062 | Oryzias latipes | Japanese Medaka | Fish | 100 | 91.6291 | Mortality | LC50 |
| 133062 | Oryzias latipes | Japanese Medaka | Fish | 100 | 91.6291 | Mortality | LC50 |
| 133062 | Oryzias latipes | Japanese Medaka | Fish | 100 | 91.6291 | Mortality | LC50 |
| 133062 | Physella acuta | European Physa | Molluscs | 100 | 91.6291 | Mortality | LC50 |
| 1912249 | Megalodiscus sp. | Fluke | Worms | 85.438 | 95.54027456 | Mortality | LC50 |
| 133062 | Daphnia magna | Water Flea | Crustaceans | 105 | 96.210555 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 87 | 97.286967 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 87 | 97.286967 | Mortality | LC50 |
| 1912249 | Scenedesmus acutus var. acutus | Green Algae | Algae | 89.75 | 100.3621298 | Mortality | LC50 |
| 133062 | Daphnia magna | Water Flea | Crustaceans | 110 | 100.79201 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 92 | 102.878172 | Mortality | LC50 |
| 50328 | Xenopus laevis | African Clawed Frog | Amphibians | 333.056196 | 107.9338545 | Mortality | LC50 |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 98 | 109.587618 | Mortality | LOEC |
| 133062 | Semisulcospira libertina | Marsh Snail | Molluscs | 120 | 109.95492 | Mortality | LC50 |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 99.85 | 111.6563639 | Mortality | LC50 |
| 1912249 | Amphiascus tenuiremis | Harpacticoid Copepod | Crustaceans | 100 | 111.8241 | Mortality | LC50 |
| 1912249 | Americamysis bahia | Opossum Shrimp | Crustaceans | 100 | 111.8241 | Mortality | LC50 |
| 1912249 | Penaeus aztecus | Brown Shrimp | Crustaceans | 100 | 111.8241 | Mortality | LC50 |
| 1912249 | Leiostomus xanthurus | Spot | Fish | 100 | 111.8241 | Mortality | LC50 |
| 1912249 | Leiostomus xanthurus | Spot | Fish | 100 | 111.8241 | Mortality | LC50 |
| 57636 | Gammarus pulex | Scud | Crustaceans | 80 | 113.5752 | Mortality | LC50 |
| 57636 | Lithobates pipiens | Leopard Frog | Amphibians | 81.512585 | 115.7226018 | Mortality | LC50 |
| 133062 | Daphnia magna | Water Flea | Crustaceans | 130 | 119.11783 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 108 | 120.770028 | Mortality | LC50 |
| 1912249 | Mesocyclops longisetus | Copepod | Crustaceans | 108.5 | 121.3291485 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 110 | 123.00651 | Mortality | LC50 |
| 1912249 | Spartina alterniflora | Smooth Cordgrass | Plants | 110.7 | 123.7892787 | Mortality | LC50 |
| 57636 | Lithobates pipiens | Leopard Frog | Amphibians | 89.2192294 | 126.6636478 | Mortality | LC50 |
| 133062 | Indoplanorbis exustus | Ram's Horn Snail | Molluscs | 140 | 128.28074 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 120 | 134.18892 | Mortality | LC50 |
| 1912249 | Salmo salar | Atlantic Salmon | Fish | 120 | 134.18892 | Mortality | NOEC |
| 133062 | Daphnia pulex | Water Flea | Crustaceans | 150 | 137.44365 | Mortality | LC50 |
| 133062 | Daphnia pulex | Water Flea | Crustaceans | 150 | 137.44365 | Mortality | LC50 |
| 133062 | Daphnia pulex | Water Flea | Crustaceans | 150 | 137.44365 | Mortality | LC50 |
| 133062 | Cloeon dipterum | Mayfly | Insects | 150 | 137.44365 | Mortality | LC50 |
| 13311847 | Neomysis integer | Opossum Shrimp | Crustaceans | 138 | 138 | Mortality | LC50 |
| 1912249 | Phoxinus phoxinus | Minnow | Fish | 125 | 139.780125 | Mortality | LC50 |
| 1912249 | Barbodes carnaticus | Carnatic Carp | Fish | 125 | 139.780125 | Mortality | LC50 |
| 1912249 | Biomphalaria alexandrina | Snail | Molluscs | 125 | 139.780125 | Mortality | LC50 |
| 57636 | Fundulus heteroclitus | Mummichog | Fish | 100 | 141.969 | Mortality | NOEC |
| 57636 | Marisa cornuarietis | Snail | Molluscs | 100 | 141.969 | Mortality | NOEC |
| 57636 | Marisa cornuarietis | Snail | Molluscs | 100 | 141.969 | Mortality | NOEC |
| 57636 | Marisa cornuarietis | Snail | Molluscs | 100 | 141.969 | Mortality | NOEC |
| 57636 | Marisa cornuarietis | Snail | Molluscs | 100 | 141.969 | Mortality | NOEC |
| 57636 | Dugesia japonica | Flatworm | Worms | 100 | 141.969 | Mortality | LC50 |
| 1912249 | Poecilia reticulata | Guppy | Fish | 129.8 | 145.1476818 | Mortality | NOEC |
| 98319267 | Oryzias latipes | Japanese Medaka | Fish | 6100 | 147.9555 | Mortality | NOEC |
| 98319267 | Oryzias latipes | Japanese Medaka | Fish | 6100 | 147.9555 | Mortality | NOEC |
| 57636 | Hyalella azteca | Scud | Crustaceans | 110 | 156.1659 | Mortality | LC50 |
| 57636 | Acartia tonsa | Calanoid Copepod | Crustaceans | 110 | 156.1659 | Mortality | LC50 |
| 1912249 | Physella acuta | European Physa | Molluscs | 141.8 | 158.5665738 | Mortality | NOEC |
| 28553120 | Americamysis bahia | Opossum Shrimp | Crustaceans | 39 | 162.239103 | Mortality | LC50 |
| 1912249 | Hyalella azteca | Scud | Crustaceans | 150 | 167.73615 | Mortality | LC50 |
| 1912249 | Mesocyclops longisetus | Copepod | Crustaceans | 150.3 | 168.0716223 | Mortality | LC50 |
| 57636 | Neomysis integer | Opossum Shrimp | Crustaceans | 120 | 170.3628 | Mortality | LC50 |
| 56531 | Xenopus laevis | African Clawed Frog | Amphibians | 26.8 | 171.6433604 | Mortality | NOEC |
| 1912249 | Spartina alterniflora | Smooth Cordgrass | Plants | 153.8 | 171.9854658 | Mortality | LC50 |
| 133062 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 190 | 174.09529 | Mortality | LC50 |
| 28553120 | Ictalurus punctatus | Channel Catfish | Fish | 42 | 174.719034 | Mortality | LC50 |
| 57636 | Lithobates pipiens | Leopard Frog | Amphibians | 123.6027198 | 175.4775453 | Mortality | LC50 |
| 56531 | Nitocra spinipes | Harpacticoid Copepod | Crustaceans | 29 | 185.733487 | Mortality | LC50 |
| 1912249 | Pseudacris regilla | Pacific Chorus Frog | Amphibians | 168.608 | 188.5443785 | Mortality | LC50 |
| 13311847 | Oryzias latipes | Japanese Medaka | Fish | 192 | 192 | Mortality | LC50 |
| 1912249 | Hyalella azteca | Scud | Crustaceans | 180 | 201.28338 | Mortality | LC50 |
| 57636 | Dugesia japonica | Flatworm | Worms | 150 | 212.9535 | Mortality | LC50 |
| 28553120 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 52 | 216.318804 | Mortality | LC50 |
| 28553120 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 52 | 216.318804 | Mortality | LC50 |
| 28553120 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 52 | 216.318804 | Mortality | LC50 |
| 28553120 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 52 | 216.318804 | Mortality | LC50 |
| 28553120 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 52 | 216.318804 | Mortality | LC50 |
| 1912249 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 200 | 223.6482 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 200 | 223.6482 | Mortality | LOEC |
| 1912249 | Marisa cornuarietis | Snail | Molluscs | 200 | 223.6482 | Mortality | LOEC |
| 1912249 | Alaria sp. | Trematode | Worms | 200 | 223.6482 | Mortality | NOEC |
| 1912249 | Echinostoma trivolvis | Trematode | Worms | 200 | 223.6482 | Mortality | NOEC |
| 57636 | Dugesia japonica | Flatworm | Worms | 165 | 234.24885 | Mortality | LC50 |
| 1912249 | Cyprinus carpio | Common Carp | Fish | 214.2 | 239.5272222 | Mortality | LC50 |
| 57636 | Danio rerio | Zebra Danio | Fish | 170 | 241.3473 | Mortality | LC50 |
| 57636 | Danio rerio | Zebra Danio | Fish | 170 | 241.3473 | Mortality | LC50 |
| 57636 | Danio rerio | Zebra Danio | Fish | 170 | 241.3473 | Mortality | LC50 |
| 57636 | Dugesia japonica | Flatworm | Worms | 170 | 241.3473 | Mortality | LC50 |
| 98319267 | Oryzias latipes | Japanese Medaka | Fish | 10200 | 247.401 | Mortality | LOEC |
| 1912249 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 230 | 257.19543 | Mortality | LC50 |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 232 | 259.431912 | Mortality | LC50 |
| 1912249 | Lithobates palustris | Pickerel Frog | Amphibians | 234 | 261.668394 | Mortality | LOEC |
| 57636 | Xenopus tropicalis | Clawed Frog | Amphibians | 190.3837576 | 270.2859169 | Mortality | NOEC |
| 57636 | Xenopus tropicalis | Clawed Frog | Amphibians | 190.3837576 | 270.2859169 | Mortality | NOEC |
| 133062 | Bufo bufo ssp. japonicus | Toad | Amphibians | 300 | 274.8873 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 300 | 274.8873 | Mortality | NOEC |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 250 | 279.56025 | Mortality | NOEC |
| 1912249 | Barbodes carnaticus | Carnatic Carp | Fish | 250 | 279.56025 | Mortality | LC50 |
| 57636 | Gammarus pulex | Scud | Crustaceans | 200 | 283.938 | Mortality | LC50 |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 200 | 283.938 | Mortality | LOEC |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 200 | 283.938 | Mortality | LOEC |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 200 | 283.938 | Mortality | LOEC |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 200 | 283.938 | Mortality | LOEC |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 260 | 290.74266 | Mortality | LC50 |
| 57636 | Chironomus tentans | Midge | Insects | 216 | 306.65304 | Mortality | LC50 |
| 1912249 | Lithobates clamitans ssp. clamitans | Bronze Frog | Amphibians | 282.3 | 315.6794343 | Mortality | NOEC |
| 50328 | Chironomus tentans | Midge | Insects | 987.3 | 319.9552983 | Mortality | LC50 |
| 56531 | Daphnia magna | Water Flea | Crustaceans | 50 | 320.23015 | Mortality | LC50 |
| 56531 | Dugesia japonica | Flatworm | Worms | 50 | 320.23015 | Mortality | LC50 |
| 28553120 | Americamysis bahia | Opossum Shrimp | Crustaceans | 77 | 320.318229 | Mortality | LC50 |
| 50328 | Xenopus laevis | African Clawed Frog | Amphibians | 1000 | 324.071 | Mortality | LC50 |
| 50328 | Xenopus laevis | African Clawed Frog | Amphibians | 1000 | 324.071 | Mortality | LC50 |
| 50328 | Xenopus laevis | African Clawed Frog | Amphibians | 1000 | 324.071 | Mortality | LC50 |
| 50328 | Xenopus laevis | African Clawed Frog | Amphibians | 1000 | 324.071 | Mortality | LC50 |
| 56531 | Dugesia japonica | Flatworm | Worms | 52 | 333.039356 | Mortality | LC50 |
| 1912249 | Ambystoma maculatum | Spotted Salamander | Amphibians | 300 | 335.4723 | Mortality | NOEC |
| 1912249 | Ambystoma maculatum | Spotted Salamander | Amphibians | 300 | 335.4723 | Mortality | NOEC |
| 1912249 | Diporeia sp. | Amphipod | Crustaceans | 300 | 335.4723 | Mortality | LC50 |
| 1912249 | Planorbella trivolvis | Snail, Marsh Rams-Horn | Molluscs | 300 | 335.4723 | Mortality | NOEC |
| 1912249 | Stagnicola elodes | Marsh Pond Snail | Molluscs | 300 | 335.4723 | Mortality | NOEC |
| 1912249 | Biomphalaria glabrata | Snail | Molluscs | 300 | 335.4723 | Mortality | NOEC |
| 1912249 | Physella acuta | European Physa | Molluscs | 300 | 335.4723 | Mortality | NOEC |
| 13311847 | Gammarus pulex | Scud | Crustaceans | 350.656 | 350.656 | Mortality | LC50 |
| 50328 | Gammarus duebeni | Scud | Crustaceans | 1100 | 356.4781 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 320 | 357.83712 | Mortality | LOEC |
| 56531 | Dugesia japonica | Flatworm | Worms | 56 | 358.657768 | Mortality | LC50 |
| 28553120 | Ictalurus punctatus | Channel Catfish | Fish | 87 | 361.917999 | Mortality | LC50 |
| 13311847 | Acartia tonsa | Calanoid Copepod | Crustaceans | 365.082 | 365.082 | Mortality | LC50 |
| 57636 | Daphnia magna | Water Flea | Crustaceans | 259.04 | 367.7564976 | Mortality | LC50 |
| 1912249 | Echinogammarus tibaldii | Amphipod | Crustaceans | 330 | 369.01953 | Mortality | LC50 |
| 133062 | Clarias batrachus | Walking Catfish | Fish | 410.36 | 376.0091748 | Mortality | LC50 |
| 56531 | Dugesia japonica | Flatworm | Worms | 60 | 384.27618 | Mortality | LC50 |
| 56531 | Dugesia japonica | Flatworm | Worms | 61 | 390.680783 | Mortality | LC50 |
| 57636 | Dugesia japonica | Flatworm | Worms | 278 | 394.67382 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 360 | 402.56676 | Mortality | LC50 |
| 90357065 | Pimephales promelas | Fathead Minnow | Fish | 921.11 | 414.5400288 | Mortality | NOEC |
| 1912249 | Phoxinus phoxinus | Minnow | Fish | 375 | 419.340375 | Mortality | LC50 |
| 57636 | Xenopus tropicalis | Clawed Frog | Amphibians | 296.4094 | 420.8094611 | Mortality | NOEC |
| 57636 | Xenopus tropicalis | Clawed Frog | Amphibians | 296.4094 | 420.8094611 | Mortality | NOEC |
| 57636 | Dugesia japonica | Flatworm | Worms | 300 | 425.907 | Mortality | LC50 |
| 50328 | Xenopus laevis | African Clawed Frog | Amphibians | 1340 | 434.25514 | Mortality | LC50 |
| 1912249 | Ambystoma barbouri | Streamside Salamander | Amphibians | 400 | 447.2964 | Mortality | NOEC |
| 1912249 | Ambystoma barbouri | Streamside Salamander | Amphibians | 400 | 447.2964 | Mortality | NOEC |
| 1912249 | Echinostoma trivolvis | Trematode | Worms | 400 | 447.2964 | Mortality | LOEC |
| 1912249 | Alaria sp. | Trematode | Worms | 400 | 447.2964 | Mortality | LOEC |
| 56531 | Dugesia japonica | Flatworm | Worms | 70 | 448.32221 | Mortality | LC50 |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 410 | 458.47881 | Mortality | LC50 |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 430 | 480.84363 | Mortality | LC50 |
| 1912249 | Poecilia reticulata | Guppy | Fish | 430 | 480.84363 | Mortality | LC50 |
| 57636 | Dugesia japonica | Flatworm | Worms | 340 | 482.6946 | Mortality | LC50 |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 436.3 | 487.8885483 | Mortality | NOEC |
| 1912249 | Gammarus pulex | Scud | Crustaceans | 440 | 492.02604 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 450 | 503.20845 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 450 | 503.20845 | Mortality | LC50 |
| 56531 | Dugesia japonica | Flatworm | Worms | 80 | 512.36824 | Mortality | LC50 |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 479 | 535.637439 | Mortality | NOEC |
| 57636 | Hydra vulgaris | Hydra | Invertebrates | 380 | 539.4822 | Mortality | LC50 |
| 13311847 | Acartia tonsa | Calanoid Copepod | Crustaceans | 540 | 540 | Mortality | LC50 |
| 50328 | Xenopus laevis | African Clawed Frog | Amphibians | 1670 | 541.19857 | Mortality | LC50 |
| 1912249 | Salvelinus fontinalis | Brook Trout | Fish | 490 | 547.93809 | Mortality | LC50 |
| 1912249 | Phoxinus phoxinus | Minnow | Fish | 500 | 559.1205 | Mortality | LC50 |
| 1912249 | Barbodes carnaticus | Carnatic Carp | Fish | 500 | 559.1205 | Mortality | LC50 |
| 1912249 | Potamopyrgus antipodarum | Snail | Molluscs | 500 | 559.1205 | Mortality | NOEC |
| 1912249 | Potamopyrgus antipodarum | Snail | Molluscs | 500 | 559.1205 | Mortality | NOEC |
| 1912249 | Potamopyrgus antipodarum | Snail | Molluscs | 500 | 559.1205 | Mortality | NOEC |
| 1912249 | Oryzias latipes | Japanese Medaka | Fish | 515 | 575.894115 | Mortality | NOEC |
| 57636 | Chironomus tentans | Midge | Insects | 410 | 582.0729 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 640 | 586.42624 | Mortality | LOEC |
| 1912249 | Lemna minor | Duckweed | Plants | 527 | 589.313007 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 530 | 592.66773 | Mortality | LC50 |
| 57636 | Gammarus pulex | Scud | Crustaceans | 419 | 594.85011 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 534.9804 | 598.2370175 | Mortality | LC50 |
| 13311847 | Dugesia japonica | Flatworm | Worms | 600 | 600 | Mortality | LC50 |
| 1912249 | Americamysis bahia | Opossum Shrimp | Crustaceans | 540 | 603.85014 | Mortality | LC50 |
| 1912249 | Rana boylii | Foothill Yellow-Legged Frog | Amphibians | 551.69 | 616.9223773 | Mortality | LC50 |
| 13311847 | Dugesia japonica | Flatworm | Worms | 620 | 620 | Mortality | LC50 |
| 133062 | Moina macrocopa | Water Flea | Crustaceans | 680 | 623.07788 | Mortality | LC50 |
| 133062 | Moina macrocopa | Water Flea | Crustaceans | 680 | 623.07788 | Mortality | LC50 |
| 133062 | Moina macrocopa | Water Flea | Crustaceans | 680 | 623.07788 | Mortality | LC50 |
| 1912249 | Mercenaria mercenaria | Northern Quahog Or Hard Clam | Molluscs | 560.8 | 627.1095528 | Mortality | LC50 |
| 1912249 | Americamysis bahia | Opossum Shrimp | Crustaceans | 570 | 637.39737 | Mortality | LC50 |
| 1912249 | Gammarus fasciatus | Scud | Crustaceans | 570 | 637.39737 | Mortality | LC50 |
| 57636 | Radix balthica | Freshwater Snail | Molluscs | 449.5 | 638.150655 | Mortality | NOEC |
| 57636 | Bithynia tentaculata | Snail | Molluscs | 449.5 | 638.150655 | Mortality | NOEC |
| 57636 | Radix balthica | Freshwater Snail | Molluscs | 449.5 | 638.150655 | Mortality | NOEC |
| 57636 | Bithynia tentaculata | Snail | Molluscs | 449.5 | 638.150655 | Mortality | NOEC |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 100 | 640.4603 | Mortality | NOEC |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 100 | 640.4603 | Mortality | NOEC |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 100 | 640.4603 | Mortality | NOEC |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 100 | 640.4603 | Mortality | NOEC |
| 1912249 | Tetrahymena pyriformis | Ciliate | Invertebrates; Standard Test Species | 583 | 651.934503 | Mortality | LC50 |
| 13311847 | Dugesia japonica | Flatworm | Worms | 660 | 660 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 603.0818 | 674.3907951 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 609 | 681.008769 | Mortality | LC50 |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 610 | 682.12701 | Mortality | LC50 |
| 56531 | Daphnia magna | Water Flea | Crustaceans | 109 | 698.101727 | Mortality | LC50 |
| 1912249 | Coregonus fera | Whitefish | Fish | 630 | 704.49183 | Mortality | LC50 |
| 1912249 | Salvelinus fontinalis | Brook Trout | Fish | 630 | 704.49183 | Mortality | LC50 |
| 1912249 | Glossiphonia complanata | Leech | Worms | 630 | 704.49183 | Mortality | LC50 |
| 1912249 | Mytilus galloprovincialis | Mediterranean Mussel | Molluscs | 633 | 707.846553 | Mortality | LC50 |
| 1689823 | Pimephales promelas | Fathead Minnow | Fish | 109 | 710.263075 | Mortality | LC50 |
| 1912249 | Americamysis bahia | Opossum Shrimp | Crustaceans | 640 | 715.67424 | Mortality | LC50 |
| 1912249 | Brachionus calyciflorus | Rotifer | Invertebrates; Standard Test Species | 640 | 715.67424 | Mortality | LOEC |
| 1689823 | Pimephales promelas | Fathead Minnow | Fish | 110 | 716.77925 | Mortality | LC50 |
| 1912249 | Paratya australiensis | Shrimp | Crustaceans | 650 | 726.85665 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 800 | 733.0328 | Mortality | LC50 |
| 1912249 | Biomphalaria alexandrina | Snail | Molluscs | 660 | 738.03906 | Mortality | LOEC |
| 1912249 | Biomphalaria alexandrina | Snail | Molluscs | 660 | 738.03906 | Mortality | LOEC |
| 1912249 | Biomphalaria alexandrina | Snail | Molluscs | 660 | 738.03906 | Mortality | LOEC |
| 1912249 | Biomphalaria alexandrina | Snail | Molluscs | 660 | 738.03906 | Mortality | LOEC |
| 1912249 | Biomphalaria alexandrina | Snail | Molluscs | 660 | 738.03906 | Mortality | LOEC |
| 28553120 | Daphnia magna | Water Flea | Crustaceans | 178 | 740.475906 | Mortality | LOEC |
| 28553120 | Daphnia magna | Water Flea | Crustaceans | 178 | 740.475906 | Mortality | LOEC |
| 1912249 | Acroneuria sp. | Stonefly | Insects | 670 | 749.22147 | Mortality | LC50 |
| 1912249 | Paratya australiensis | Shrimp | Crustaceans | 680 | 760.40388 | Mortality | LC50 |
| 1689823 | Pimephales promelas | Fathead Minnow | Fish | 117 | 762.392475 | Mortality | LC50 |
| 56531 | Daphnia magna | Water Flea | Crustaceans | 120 | 768.55236 | Mortality | LC50 |
| 133062 | Americamysis bahia | Opossum Shrimp | Crustaceans | 840 | 769.68444 | Mortality | LC50 |
| 1912249 | Farfantepenaeus duorarum | Northern Pink Shrimp | Crustaceans | 690 | 771.58629 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 690 | 771.58629 | Mortality | LC50 |
| 57636 | Hydra vulgaris | Hydra | Invertebrates | 550 | 780.8295 | Mortality | LC50 |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 703 | 786.123423 | Mortality | LC50 |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 715 | 799.542315 | Mortality | LC50 |
| 1912249 | Oreochromis niloticus | Nile Tilapia | Fish | 720 | 805.13352 | Mortality | LC50 |
| 1912249 | Oreochromis niloticus | Nile Tilapia | Fish | 730 | 816.31593 | Mortality | LC50 |
| 1912249 | Robertsonia propinqua | Marine Harpacticoid Copepod | Crustaceans | 750 | 838.68075 | Mortality | LC50 |
| 1912249 | Americamysis bahia | Opossum Shrimp | Crustaceans | 750 | 838.68075 | Mortality | LC50 |
| 1912249 | Gammarus fossarum | Scud | Crustaceans | 750 | 838.68075 | Mortality | LC50 |
| 1912249 | Gammarus fossarum | Scud | Crustaceans | 750 | 838.68075 | Mortality | LC50 |
| 1912249 | Barbodes carnaticus | Carnatic Carp | Fish | 750 | 838.68075 | Mortality | LC50 |
| 1912249 | Robertsonia propinqua | Marine Harpacticoid Copepod | Crustaceans | 758 | 847.626678 | Mortality | LC50 |
| 1912249 | Oreochromis niloticus | Nile Tilapia | Fish | 760 | 849.86316 | Mortality | LC50 |
| 1912249 | Ameiurus melas | Black Bullhead | Fish | 760 | 849.86316 | Mortality | LC50 |
| 57636 | Ceriodaphnia reticulata | Water Flea | Crustaceans | 600 | 851.814 | Mortality | LOEC |
| 1912249 | Brachionus calyciflorus | Rotifer | Invertebrates; Standard Test Species | 784 | 876.700944 | Mortality | LC50 |
| 1912249 | Ceriodaphnia reticulata | Water Flea | Crustaceans | 786 | 878.937426 | Mortality | LC50 |
| 1912249 | Oreochromis niloticus | Nile Tilapia | Fish | 790 | 883.41039 | Mortality | LC50 |
| 1912249 | Acartia clausi | Calanoid Copepod | Crustaceans | 794.5 | 888.4424745 | Mortality | LC50 |
| 1912249 | Ambystoma barbouri | Streamside Salamander | Amphibians | 800 | 894.5928 | Mortality | LOEC |
| 1912249 | Ambystoma barbouri | Streamside Salamander | Amphibians | 800 | 894.5928 | Mortality | LOEC |
| 1912249 | Ameiurus melas | Black Bullhead | Fish | 800 | 894.5928 | Mortality | LC50 |
| 1912249 | Lepomis macrochirus | Bluegill | Fish | 800 | 894.5928 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 800 | 894.5928 | Mortality | NOEC |
| 1912249 | Brachionus calyciflorus | Rotifer | Invertebrates; Standard Test Species | 800 | 894.5928 | Mortality | NOEC |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 1000 | 916.291 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 1000 | 916.291 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 1000 | 916.291 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 1000 | 916.291 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 1000 | 916.291 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 1000 | 916.291 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 1000 | 916.291 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 1000 | 916.291 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 1000 | 916.291 | Mortality | LC50 |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 840 | 939.32244 | Mortality | LC50 |
| 1912249 | Leiostomus xanthurus | Spot | Fish | 850 | 950.50485 | Mortality | LC50 |
| 57636 | Hydra vulgaris | Hydra | Invertebrates | 670 | 951.1923 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 880 | 984.05208 | Mortality | LC50 |
| 57636 | Hyalella azteca | Scud | Crustaceans | 700 | 993.783 | Mortality | NOEC |
| 1912249 | Palaemonetes argentinus | Caridean Shrimp | Crustaceans | 890 | 995.23449 | Mortality | LC50 |
| 1912249 | Palaemon pugio | Daggerblade Grass Shrimp | Crustaceans | 900 | 1006.4169 | Mortality | LC50 |
| 50328 | Chironomus riparius | Midge | Insects | 3159 | 1023.740289 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1120 | 1026.24592 | Mortality | LOEC |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 940 | 1051.14654 | Mortality | LC50 |
| 1689823 | Pimephales promelas | Fathead Minnow | Fish | 164 | 1068.6527 | Mortality | LC50 |
| 1689823 | Pimephales promelas | Fathead Minnow | Fish | 164 | 1068.6527 | Mortality | LC50 |
| 1912249 | Xenopus tropicalis | Clawed Frog | Amphibians | 962 | 1075.747842 | Mortality | LC50 |
| 1912249 | Paratya australiensis | Shrimp | Crustaceans | 970 | 1084.69377 | Mortality | LC50 |
| 1912249 | Paratya australiensis | Shrimp | Crustaceans | 980 | 1095.87618 | Mortality | LC50 |
| 1912249 | Paratya australiensis | Shrimp | Crustaceans | 990 | 1107.05859 | Mortality | LC50 |
| 1912249 | Helobdella stagnalis | Leech | Worms | 990 | 1107.05859 | Mortality | LC50 |
| 1912249 | Psetta maxima | Left-Eyed Flounder, Turbot | Fish | 995.7 | 1113.432564 | Mortality | LC50 |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 1000 | 1118.241 | Mortality | NOEC |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 1000 | 1118.241 | Mortality | NOEC |
| 1912249 | Palaemon pugio | Daggerblade Grass Shrimp | Crustaceans | 1000 | 1118.241 | Mortality | LC50 |
| 1912249 | Carcinus maenas | Green Crab | Crustaceans | 1000 | 1118.241 | Mortality | LC50 |
| 1912249 | Poecilia reticulata | Guppy | Fish | 1000 | 1118.241 | Mortality | LC50 |
| 1912249 | Cyprinus carpio | Common Carp | Fish | 1000 | 1118.241 | Mortality | LC50 |
| 1912249 | Carassius auratus | Goldfish | Fish | 1000 | 1118.241 | Mortality | LC50 |
| 1912249 | Carassius auratus | Goldfish | Fish | 1000 | 1118.241 | Mortality | LC50 |
| 1912249 | Oryzias latipes | Japanese Medaka | Fish | 1000 | 1118.241 | Mortality | LC50 |
| 1912249 | Cyprinus carpio | Common Carp | Fish | 1000 | 1118.241 | Mortality | LC50 |
| 1912249 | Oryzias latipes | Japanese Medaka | Fish | 1000 | 1118.241 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 1000 | 1118.241 | Mortality | NOEC |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1000 | 1118.241 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1000 | 1118.241 | Mortality | LC50 |
| 1912249 | Chironomus riparius | Midge | Insects | 1000 | 1118.241 | Mortality | EC50 |
| 57636 | Gammarus pulex | Scud | Crustaceans | 792 | 1124.39448 | Mortality | LC50 |
| 1912249 | Gammarus italicus | Scud | Crustaceans | 1010 | 1129.42341 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 1010 | 1129.42341 | Mortality | LC50 |
| 1912249 | Ceriodaphnia dubia | Water Flea | Crustaceans | 1016.2 | 1136.356504 | Mortality | NOEC |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 1016.2 | 1136.356504 | Mortality | NOEC |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1020 | 1140.60582 | Mortality | LC50 |
| 1912249 | Rhamdia quelen | Catfish | Fish | 1020 | 1140.60582 | Mortality | LC50 |
| 57636 | Lithobates sylvaticus | Wood Frog | Amphibians | 809 | 1148.52921 | Mortality | NOEC |
| 1912249 | Macrothrix flabelligera | Water Flea | Crustaceans | 1052 | 1176.389532 | Mortality | LC50 |
| 1912249 | Bufo americanus | American Toad | Amphibians | 1070 | 1196.51787 | Mortality | LC50 |
| 57636 | Hydra vulgaris | Hydra | Invertebrates | 860 | 1220.9334 | Mortality | LC50 |
| 28553120 | Bufo woodhousei ssp. fowleri | Fowler's Toad | Amphibians | 295 | 1227.193215 | Mortality | LC50 |
| 1912249 | Xenopus tropicalis | Clawed Frog | Amphibians | 1100 | 1230.0651 | Mortality | LC50 |
| 1912249 | Macrothrix flabelligera | Water Flea | Crustaceans | 1115 | 1246.838715 | Mortality | LC50 |
| 1912249 | Coregonus fera | Whitefish | Fish | 1120 | 1252.42992 | Mortality | LC50 |
| 57636 | Chironomus riparius | Midge | Insects | 883 | 1253.58627 | Mortality | LC50 |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 200 | 1280.9206 | Mortality | LOEC |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 200 | 1280.9206 | Mortality | LOEC |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 200 | 1280.9206 | Mortality | LOEC |
| 56531 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 200 | 1280.9206 | Mortality | LOEC |
| 57636 | Chironomus riparius | Midge | Insects | 913.6 | 1297.028784 | Mortality | LC50 |
| 1912249 | Lithobates clamitans ssp. clamitans | Bronze Frog | Amphibians | 1170 | 1308.34197 | Mortality | NOEC |
| 1912249 | Psetta maxima | Left-Eyed Flounder, Turbot | Fish | 1187.3 | 1327.687539 | Mortality | LC50 |
| 1912249 | Oncorhynchus kisutch | Silver Salmon | Fish | 1200 | 1341.8892 | Mortality | LC50 |
| 1912249 | Oncorhynchus kisutch | Silver Salmon | Fish | 1200 | 1341.8892 | Mortality | LC50 |
| 13311847 | Dugesia japonica | Flatworm | Worms | 1360 | 1360 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1230 | 1375.43643 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1260 | 1408.98366 | Mortality | LC50 |
| 1912249 | Ceriodaphnia silvestrii | Water Flea | Crustaceans | 1262 | 1411.220142 | Mortality | LC50 |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 1263 | 1412.338383 | Mortality | LC50 |
| 28553120 | Daphnia magna | Water Flea | Crustaceans | 340 | 1414.39218 | Mortality | NOEC |
| 28553120 | Daphnia magna | Water Flea | Crustaceans | 340 | 1414.39218 | Mortality | NOEC |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 1000 | 1419.69 | Mortality | NOEC |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 1000 | 1419.69 | Mortality | NOEC |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 1000 | 1419.69 | Mortality | NOEC |
| 57636 | Tisbe battagliai | Harpacticoid Copepod | Crustaceans | 1000 | 1419.69 | Mortality | NOEC |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1280 | 1431.34848 | Mortality | LC50 |
| 1912249 | Xenopus tropicalis | Clawed Frog | Amphibians | 1290 | 1442.53089 | Mortality | LC50 |
| 1912249 | Hyalella azteca | Scud | Crustaceans | 1300 | 1453.7133 | Mortality | LC50 |
| 1912249 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 1300 | 1453.7133 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1300 | 1453.7133 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1300 | 1453.7133 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1300 | 1453.7133 | Mortality | LC50 |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 1320 | 1476.07812 | Mortality | LC50 |
| 1912249 | Quinquelaophonte sp. | Copepod | Crustaceans | 1330 | 1487.26053 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1340 | 1498.44294 | Mortality | LC50 |
| 1912249 | Gammarus pulex | Scud | Crustaceans | 1340 | 1498.44294 | Mortality | LC50 |
| 1912249 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 1340 | 1498.44294 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1350 | 1509.62535 | Mortality | LC50 |
| 28553120 | Lithobates pipiens | Leopard Frog | Amphibians | 363 | 1510.071651 | Mortality | LC50 |
| 1912249 | Oncorhynchus kisutch | Silver Salmon | Fish | 1400 | 1565.5374 | Mortality | LC50 |
| 1912249 | Oncorhynchus kisutch | Silver Salmon | Fish | 1400 | 1565.5374 | Mortality | LC50 |
| 1912249 | Oncorhynchus nerka | Sockeye Salmon | Fish | 1410 | 1576.71981 | Mortality | NOEC |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 1441 | 1611.385281 | Mortality | LC50 |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 1443 | 1613.621763 | Mortality | LC50 |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 1448 | 1619.212968 | Mortality | LC50 |
| 1912249 | Lithobates pipiens | Leopard Frog | Amphibians | 1450 | 1621.44945 | Mortality | LC50 |
| 1912249 | Ceriodaphnia silvestrii | Water Flea | Crustaceans | 1460 | 1632.63186 | Mortality | LC50 |
| 1912249 | Hyalella azteca | Scud | Crustaceans | 1470 | 1643.81427 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1470 | 1643.81427 | Mortality | LC50 |
| 133062 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1800 | 1649.3238 | Mortality | NOEC |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 1475 | 1649.405475 | Mortality | LC50 |
| 1912249 | Gammarus pulex | Scud | Crustaceans | 1490 | 1666.17909 | Mortality | LC50 |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 1500 | 1677.3615 | Mortality | LC50 |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 1500 | 1677.3615 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1500 | 1677.3615 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1520 | 1699.72632 | Mortality | LC50 |
| 1912249 | Macrothrix flabelligera | Water Flea | Crustaceans | 1543 | 1725.445863 | Mortality | LC50 |
| 1912249 | Notropis atherinoides | Emerald Shiner | Fish | 1560 | 1744.45596 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 1563 | 1747.810683 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 1563 | 1747.810683 | Mortality | LC50 |
| 1912249 | Ceriodaphnia silvestrii | Water Flea | Crustaceans | 1567 | 1752.283647 | Mortality | LC50 |
| 1912249 | Bufo americanus | American Toad | Amphibians | 1580 | 1766.82078 | Mortality | LC50 |
| 1912249 | Lithobates catesbeianus | American Bullfrog | Amphibians | 1600 | 1789.1856 | Mortality | LC50 |
| 1912249 | Lithobates catesbeianus | American Bullfrog | Amphibians | 1600 | 1789.1856 | Mortality | LC50 |
| 1912249 | Lithobates catesbeianus | American Bullfrog | Amphibians | 1600 | 1789.1856 | Mortality | LC50 |
| 1912249 | Lithobates catesbeianus | American Bullfrog | Amphibians | 1600 | 1789.1856 | Mortality | LC50 |
| 1912249 | Perca sp. | Perch | Fish | 1600 | 1789.1856 | Mortality | LC50 |
| 1912249 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 1600 | 1789.1856 | Mortality | LC50 |
| 1912249 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 1620 | 1811.55042 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1640 | 1833.91524 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1640 | 1833.91524 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1640 | 1833.91524 | Mortality | LC50 |
| 1912249 | Chironomus tepperi | Rice Bloodworm | Insects | 1660 | 1856.28006 | Mortality | LC50 |
| 1912249 | Chironomus tepperi | Rice Bloodworm | Insects | 1670 | 1867.46247 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 1682.34846 | 1881.271024 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1690 | 1889.82729 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1690 | 1889.82729 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1690 | 1889.82729 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1700 | 1901.0097 | Mortality | LC50 |
| 56531 | Nitocra spinipes | Harpacticoid Copepod | Crustaceans | 300 | 1921.3809 | Mortality | NOEC |
| 28553120 | Lepomis microlophus | Redear Sunfish | Fish | 467 | 1942.709259 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1740 | 1945.73934 | Mortality | LC50 |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 1827 | 2043.026307 | Mortality | LC50 |
| 1912249 | Chironomus tepperi | Rice Bloodworm | Insects | 1830 | 2046.38103 | Mortality | LC50 |
| 28553120 | Lithobates pipiens | Leopard Frog | Amphibians | 494 | 2055.028638 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1840 | 2057.56344 | Mortality | LC50 |
| 1912249 | Cyprinus carpio | Common Carp | Fish | 1850 | 2068.74585 | Mortality | LC50 |
| 1912249 | Rutilus kutum | Kutum | Fish | 1853 | 2072.100573 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1870 | 2091.11067 | Mortality | LC50 |
| 57636 | Hyalella azteca | Scud | Crustaceans | 1480 | 2101.1412 | Mortality | LOEC |
| 1912249 | Cyprinus carpio | Common Carp | Fish | 1880 | 2102.29308 | Mortality | LC50 |
| 1912249 | Gammarus kischineffensis | Amphipod | Crustaceans | 1890 | 2113.47549 | Mortality | LC50 |
| 1912249 | Chironomus riparius | Midge | Insects | 1890 | 2113.47549 | Mortality | LC50 |
| 1912249 | Oncorhynchus tshawytscha | Chinook Salmon | Fish | 1900 | 2124.6579 | Mortality | LC50 |
| 1912249 | Xenopus tropicalis | Clawed Frog | Amphibians | 1920 | 2147.02272 | Mortality | LC50 |
| 1912249 | Lithobates pipiens | Leopard Frog | Amphibians | 1921.8 | 2149.035554 | Mortality | NOEC |
| 1912249 | Xenopus tropicalis | Clawed Frog | Amphibians | 1940 | 2169.38754 | Mortality | LC50 |
| 1912249 | Physalaemus cuvieri | Cuvier's Foam Froglet | Amphibians | 1969 | 2201.816529 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1970 | 2202.93477 | Mortality | LC50 |
| 1912249 | Rutilus kutum | Kutum | Fish | 1977 | 2210.762457 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 1990 | 2225.29959 | Mortality | LC50 |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 2000 | 2236.482 | Mortality | LOEC |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 2000 | 2236.482 | Mortality | LOEC |
| 1912249 | Ambystoma tigrinum | Tiger Salamander | Amphibians | 2000 | 2236.482 | Mortality | NOEC |
| 1912249 | Lithobates pipiens | Leopard Frog | Amphibians | 2000 | 2236.482 | Mortality | NOEC |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 2000 | 2236.482 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 2000 | 2236.482 | Mortality | LOEC |
| 1912249 | Oncorhynchus tshawytscha | Chinook Salmon | Fish | 2000 | 2236.482 | Mortality | LC50 |
| 1912249 | Oncorhynchus tshawytscha | Chinook Salmon | Fish | 2000 | 2236.482 | Mortality | LC50 |
| 1912249 | Elodea canadensis | Waterweed | Plants | 2000 | 2236.482 | Mortality | EC50 |
| 1912249 | Biomphalaria glabrata | Snail | Molluscs | 2000 | 2236.482 | Mortality | LOEC |
| 1912249 | Stagnicola elodes | Marsh Pond Snail | Molluscs | 2000 | 2236.482 | Mortality | LOEC |
| 1912249 | Physella acuta | European Physa | Molluscs | 2000 | 2236.482 | Mortality | LOEC |
| 1912249 | Planorbella trivolvis | Snail, Marsh Rams-Horn | Molluscs | 2000 | 2236.482 | Mortality | LOEC |
| 1912249 | Mercenaria mercenaria | Northern Quahog Or Hard Clam | Molluscs | 2000 | 2236.482 | Mortality | LOEC |
| 1912249 | Megalodiscus sp. | Fluke | Worms | 2000 | 2236.482 | Mortality | NOEC |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 2011 | 2248.782651 | Mortality | LC50 |
| 1912249 | Chironomus tepperi | Rice Bloodworm | Insects | 2020 | 2258.84682 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 2050 | 2292.39405 | Mortality | LC50 |
| 1912249 | Quinquelaophonte sp. | Copepod | Crustaceans | 2080 | 2325.94128 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 2090 | 2337.12369 | Mortality | LC50 |
| 1912249 | Perca sp. | Perch | Fish | 2100 | 2348.3061 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 2160 | 2415.40056 | Mortality | LC50 |
| 1912249 | Rutilus kutum | Kutum | Fish | 2178 | 2435.528898 | Mortality | LC50 |
| 1912249 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 2200 | 2460.1302 | Mortality | LC50 |
| 1912249 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 2200 | 2460.1302 | Mortality | LC50 |
| 1912249 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 2200 | 2460.1302 | Mortality | LC50 |
| 1912249 | Oncorhynchus tshawytscha | Chinook Salmon | Fish | 2200 | 2460.1302 | Mortality | LC50 |
| 1912249 | Acartia clausi | Calanoid Copepod | Crustaceans | 2229 | 2492.559189 | Mortality | LC50 |
| 1912249 | Xenopus tropicalis | Clawed Frog | Amphibians | 2230 | 2493.67743 | Mortality | LC50 |
| 1912249 | Coregonus fera | Whitefish | Fish | 2240 | 2504.85984 | Mortality | LC50 |
| 1912249 | Rutilus kutum | Kutum | Fish | 2243 | 2508.214563 | Mortality | LC50 |
| 1912249 | Acartia clausi | Calanoid Copepod | Crustaceans | 2247 | 2512.687527 | Mortality | LC50 |
| 1912249 | Rutilus kutum | Kutum | Fish | 2312 | 2585.373192 | Mortality | LC50 |
| 1912249 | Gammarus kischineffensis | Amphipod | Crustaceans | 2340 | 2616.68394 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 2350 | 2627.86635 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 2900 | 2657.2439 | Mortality | NOEC |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 2900 | 2657.2439 | Mortality | NOEC |
| 1912249 | Ictalurus punctatus | Channel Catfish | Fish | 2380 | 2661.41358 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 2380 | 2661.41358 | Mortality | LC50 |
| 28553120 | Daphnia magna | Water Flea | Crustaceans | 640 | 2662.38528 | Mortality | LOEC |
| 1912249 | Lepomis macrochirus | Bluegill | Fish | 2399.999 | 2683.777282 | Mortality | LC50 |
| 1912249 | Lepomis macrochirus | Bluegill | Fish | 2400 | 2683.7784 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 2400 | 2683.7784 | Mortality | LC50 |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 2450 | 2739.69045 | Mortality | LC50 |
| 1912249 | Amphiascus tenuiremis | Harpacticoid Copepod | Crustaceans | 2466 | 2757.582306 | Mortality | NOEC |
| 1912249 | Amphiascus tenuiremis | Harpacticoid Copepod | Crustaceans | 2466 | 2757.582306 | Mortality | NOEC |
| 1912249 | Amphiascus tenuiremis | Harpacticoid Copepod | Crustaceans | 2466 | 2757.582306 | Mortality | NOEC |
| 1912249 | Amphiascus tenuiremis | Harpacticoid Copepod | Crustaceans | 2466 | 2757.582306 | Mortality | NOEC |
| 1912249 | Coregonus fera | Whitefish | Fish | 2470 | 2762.05527 | Mortality | LC50 |
| 1912249 | Rutilus kutum | Kutum | Fish | 2495 | 2790.011295 | Mortality | LC50 |
| 1912249 | Psetta maxima | Left-Eyed Flounder, Turbot | Fish | 2500 | 2795.6025 | Mortality | LOEC |
| 1912249 | Salmo trutta | Brown Trout | Fish | 2500 | 2795.6025 | Mortality | LC50 |
| 1912249 | Mercenaria mercenaria | Northern Quahog Or Hard Clam | Molluscs | 2500 | 2795.6025 | Mortality | LOEC |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 2537 | 2836.977417 | Mortality | LC50 |
| 57636 | Ceriodaphnia reticulata | Water Flea | Crustaceans | 2000 | 2839.38 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 2560 | 2862.69696 | Mortality | LC50 |
| 1912249 | Rutilus kutum | Kutum | Fish | 2578 | 2882.825298 | Mortality | LC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 2600 | 2907.4266 | Mortality | LC50 |
| 1912249 | Lepomis macrochirus | Bluegill | Fish | 2600 | 2907.4266 | Mortality | LC50 |
| 1912249 | Coregonus fera | Whitefish | Fish | 2630 | 2940.97383 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 2640 | 2952.15624 | Mortality | LC50 |
| 1912249 | Piaractus mesopotamicus | Pacu | Fish | 2646 | 2958.865686 | Mortality | LC50 |
| 1912249 | Bufo americanus | American Toad | Amphibians | 2650 | 2963.33865 | Mortality | LC50 |
| 1912249 | Gammarus kischineffensis | Amphipod | Crustaceans | 2680 | 2996.88588 | Mortality | LC50 |
| 1912249 | Salmo trutta | Brown Trout | Fish | 2700 | 3019.2507 | Mortality | LC50 |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 2716 | 3037.142556 | Mortality | LC50 |
| 1912249 | Rutilus kutum | Kutum | Fish | 2727 | 3049.443207 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 2737 | 3060.625617 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 2737 | 3060.625617 | Mortality | LC50 |
| 1912249 | Xenopus tropicalis | Clawed Frog | Amphibians | 2750 | 3075.16275 | Mortality | LC50 |
| 1912249 | Chironomus tentans | Midge | Insects | 2800 | 3131.0748 | Mortality | LC50 |
| 1912249 | Chironomus tentans | Midge | Insects | 2800 | 3131.0748 | Mortality | LC50 |
| 1912249 | Chironomus tentans | Midge | Insects | 2800 | 3131.0748 | Mortality | LC50 |
| 1912249 | Chironomus tentans | Midge | Insects | 2800 | 3131.0748 | Mortality | LC50 |
| 1912249 | Rutilus kutum | Kutum | Fish | 2844 | 3180.277404 | Mortality | LC50 |
| 13311847 | Oryzias latipes | Japanese Medaka | Fish | 3200 | 3200 | Mortality | NOEC |
| 1912249 | Uca pugilator | Fiddler Crab | Crustaceans | 2900 | 3242.8989 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 2906 | 3249.608346 | Mortality | LC50 |
| 1912249 | Rutilus kutum | Kutum | Fish | 2922 | 3267.500202 | Mortality | LC50 |
| 1806264 | Xenopus laevis | African Clawed Frog | Amphibians | 20.63281 | 3268.249484 | Mortality | NOEC |
| 1912249 | Coregonus fera | Whitefish | Fish | 2950 | 3298.81095 | Mortality | LC50 |
| 1912249 | Streptocephalus rubricaudatus | Fairy Shrimp | Crustaceans | 3000 | 3354.723 | Mortality | LC50 |
| 1912249 | Streptocephalus texanus | Fairy Shrimp | Crustaceans | 3000 | 3354.723 | Mortality | LC50 |
| 1912249 | Artemia salina | Brine Shrimp | Crustaceans | 3000 | 3354.723 | Mortality | LC50 |
| 1912249 | Ceriodaphnia dubia | Water Flea | Crustaceans | 3000 | 3354.723 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 3000 | 3354.723 | Mortality | NOEC |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 3000 | 3354.723 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 3074 | 3437.472834 | Mortality | LC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 3100 | 3466.5471 | Mortality | EC50 |
| 1912249 | Poecilia reticulata | Guppy | Fish | 3160 | 3533.64156 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 3160 | 3533.64156 | Mortality | LC50 |
| 1912249 | Xenopus tropicalis | Clawed Frog | Amphibians | 3180 | 3556.00638 | Mortality | LC50 |
| 1912249 | Robertsonia propinqua | Marine Harpacticoid Copepod | Crustaceans | 3180 | 3556.00638 | Mortality | LC50 |
| 28553120 | Daphnia magna | Water Flea | Crustaceans | 860 | 3577.58022 | Mortality | NOEC |
| 1912249 | Coenagrion puella | Azure Damselfly | Insects | 3231 | 3613.036671 | Mortality | NOEC |
| 1912249 | Hyalella azteca | Scud | Crustaceans | 3300 | 3690.1953 | Mortality | LC50 |
| 1912249 | Leuciscus idus ssp. melanotus | Carp | Fish | 3300 | 3690.1953 | Mortality | LC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 3300 | 3690.1953 | Mortality | LC50 |
| 1912249 | Biomphalaria alexandrina | Snail | Molluscs | 3300 | 3690.1953 | Mortality | NOEC |
| 1912249 | Coregonus fera | Whitefish | Fish | 3330 | 3723.74253 | Mortality | LC50 |
| 1912249 | Ambystoma maculatum | Spotted Salamander | Amphibians | 3380 | 3779.65458 | Mortality | NOEC |
| 1912249 | Ambystoma maculatum | Spotted Salamander | Amphibians | 3380 | 3779.65458 | Mortality | NOEC |
| 1912249 | Paratya australiensis | Shrimp | Crustaceans | 3400 | 3802.0194 | Mortality | NOEC |
| 1912249 | Pseudaphritis urvillii | Blennie | Fish | 3400 | 3802.0194 | Mortality | NOEC |
| 1912249 | Galaxias maculatus | Common Jollytail | Fish | 3400 | 3802.0194 | Mortality | NOEC |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 3400 | 3802.0194 | Mortality | NOEC |
| 1912249 | Physa virgata | Snail | Molluscs | 3410 | 3813.20181 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 3419 | 3823.265979 | Mortality | LC50 |
| 1912249 | Ameiurus melas | Black Bullhead | Fish | 3500 | 3913.8435 | Mortality | LC50 |
| 1912249 | Gammarus kischineffensis | Amphipod | Crustaceans | 3550 | 3969.75555 | Mortality | LC50 |
| 1912249 | Coregonus fera | Whitefish | Fish | 3550 | 3969.75555 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 3600 | 4025.6676 | Mortality | LC50 |
| 1912249 | Thamnocephalus platyurus | Fairy Shrimp | Crustaceans | 3670 | 4103.94447 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 3680 | 4115.12688 | Mortality | LC50 |
| 1912249 | Ameiurus melas | Black Bullhead | Fish | 3700 | 4137.4917 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 3700 | 4137.4917 | Mortality | LC50 |
| 1912249 | Oncorhynchus tshawytscha | Chinook Salmon | Fish | 3700 | 4137.4917 | Mortality | LC50 |
| 1912249 | Ctenopharyngodon idella | Grass Carp | Fish | 3700 | 4137.4917 | Mortality | LC50 |
| 1912249 | Lumbriculus variegatus | Oligochaete, Worm | Worms | 3710 | 4148.67411 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 3800 | 4249.3158 | Mortality | LC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 3800 | 4249.3158 | Mortality | LOEC |
| 1912249 | Poecilia reticulata | Guppy | Fish | 3820 | 4271.68062 | Mortality | LC50 |
| 1912249 | Tetrahymena pyriformis | Ciliate | Invertebrates; Standard Test Species | 3840 | 4294.04544 | Mortality | LC50 |
| 1912249 | Brachionus calyciflorus | Rotifer | Invertebrates; Standard Test Species | 3920 | 4383.50472 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 3951 | 4418.170191 | Mortality | LC50 |
| 1912249 | Moina macrocopa | Water Flea | Crustaceans | 4000 | 4472.964 | Mortality | LC50 |
| 1912249 | Moina macrocopa | Water Flea | Crustaceans | 4000 | 4472.964 | Mortality | LC50 |
| 1912249 | Daphnia pulex | Water Flea | Crustaceans | 4000 | 4472.964 | Mortality | LC50 |
| 1912249 | Daphnia pulex | Water Flea | Crustaceans | 4000 | 4472.964 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 4000 | 4472.964 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 4098 | 4582.551618 | Mortality | LC50 |
| 1912249 | Cyprinus carpio | Common Carp | Fish | 4100 | 4584.7881 | Mortality | LC50 |
| 1912249 | Cyprinus carpio | Common Carp | Fish | 4100 | 4584.7881 | Mortality | LC50 |
| 1912249 | Oncorhynchus tshawytscha | Chinook Salmon | Fish | 4100 | 4584.7881 | Mortality | LC50 |
| 1912249 | Artemia salina | Brine Shrimp | Crustaceans | 4178 | 4672.010898 | Mortality | LC50 |
| 1912249 | Lepomis macrochirus | Bluegill | Fish | 4200 | 4696.6122 | Mortality | LC50 |
| 1912249 | Channa punctata | Snake-Head Catfish | Fish | 4238 | 4739.105358 | Mortality | LC50 |
| 1912249 | Channa punctata | Snake-Head Catfish | Fish | 4238.1 | 4739.217182 | Mortality | LC50 |
| 1912249 | Asellus aquaticus | Aquatic Sowbug | Crustaceans | 4250 | 4752.52425 | Mortality | LC50 |
| 1912249 | Asellus aquaticus | Aquatic Sowbug | Crustaceans | 4250 | 4752.52425 | Mortality | LC50 |
| 1912249 | Oncorhynchus kisutch | Silver Salmon | Fish | 4300 | 4808.4363 | Mortality | LC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 4300 | 4808.4363 | Mortality | EC50 |
| 1912249 | Quinquelaophonte sp. | Copepod | Crustaceans | 4400 | 4920.2604 | Mortality | LC50 |
| 1912249 | Leuciscus idus | Ide, Silver Or Golden Orfe | Fish | 4400 | 4920.2604 | Mortality | LC50 |
| 1912249 | Coregonus fera | Whitefish | Fish | 4400 | 4920.2604 | Mortality | LC50 |
| 1912249 | Channa punctata | Snake-Head Catfish | Fish | 4441.2 | 4966.331929 | Mortality | LC50 |
| 1912249 | Lithobates pipiens | Leopard Frog | Amphibians | 4530 | 5065.63173 | Mortality | LC50 |
| 1912249 | Oncorhynchus tshawytscha | Chinook Salmon | Fish | 4600 | 5143.9086 | Mortality | LC50 |
| 1912249 | Oncorhynchus kisutch | Silver Salmon | Fish | 4700 | 5255.7327 | Mortality | LC50 |
| 1912249 | Lithobates pipiens | Leopard Frog | Amphibians | 4760 | 5322.82716 | Mortality | LC50 |
| 1912249 | Bufo americanus | American Toad | Amphibians | 4800 | 5367.5568 | Mortality | LC50 |
| 1912249 | Americamysis bahia | Opossum Shrimp | Crustaceans | 4800 | 5367.5568 | Mortality | LOEC |
| 1912249 | Lepomis macrochirus | Bluegill | Fish | 4800 | 5367.5568 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 4800 | 5367.5568 | Mortality | LC50 |
| 1912249 | Oncorhynchus kisutch | Silver Salmon | Fish | 4800 | 5367.5568 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 4860 | 5434.65126 | Mortality | LC50 |
| 1912249 | Channa punctata | Snake-Head Catfish | Fish | 4910 | 5490.56331 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 4918 | 5499.509238 | Mortality | LC50 |
| 1912249 | Perca sp. | Perch | Fish | 5000 | 5591.205 | Mortality | LC50 |
| 1912249 | Psetta maxima | Left-Eyed Flounder, Turbot | Fish | 5000 | 5591.205 | Mortality | LOEC |
| 1912249 | Cyprinus carpio | Common Carp | Fish | 5000 | 5591.205 | Mortality | LC50 |
| 1912249 | Oncorhynchus kisutch | Silver Salmon | Fish | 5000 | 5591.205 | Mortality | LC50 |
| 1912249 | Mercenaria mercenaria | Northern Quahog Or Hard Clam | Molluscs | 5000 | 5591.205 | Mortality | NOEC |
| 1912249 | Ictalurus punctatus | Channel Catfish | Fish | 5130 | 5736.57633 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 5344 | 5975.879904 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 5400 | 6038.5014 | Mortality | LC50 |
| 1912249 | Lepomis macrochirus | Bluegill | Fish | 5451 | 6095.531691 | Mortality | LC50 |
| 1912249 | Rasbora heteromorpha | Harlequinfish, Red Rasbora | Fish | 5520 | 6172.69032 | Mortality | LC50 |
| 1912249 | Carassius auratus | Goldfish | Fish | 5599.996 | 6262.145127 | Mortality | LC50 |
| 1912249 | Carassius auratus | Goldfish | Fish | 5600 | 6262.1496 | Mortality | LC50 |
| 13311847 | Oryzias latipes | Japanese Medaka | Fish | 6400 | 6400 | Mortality | LOEC |
| 56531 | Daphnia magna | Water Flea | Crustaceans | 1000 | 6404.603 | Mortality | NOEC |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 5800 | 6485.7978 | Mortality | LOEC |
| 1912249 | Carassius auratus | Goldfish | Fish | 5861.535 | 6554.60876 | Mortality | LC50 |
| 1912249 | Carassius auratus | Goldfish | Fish | 6000 | 6709.446 | Mortality | LC50 |
| 1912249 | Carassius auratus | Goldfish | Fish | 6000 | 6709.446 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 6000 | 6709.446 | Mortality | LOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 6000 | 6709.446 | Mortality | LOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 6000 | 6709.446 | Mortality | LOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 6000 | 6709.446 | Mortality | LOEC |
| 1912249 | Anodonta imbecillis | Mussel | Molluscs | 6000 | 6709.446 | Mortality | LC50 |
| 1912249 | Anodonta imbecillis | Mussel | Molluscs | 6000 | 6709.446 | Mortality | LC50 |
| 1912249 | Anodonta imbecillis | Mussel | Molluscs | 6000 | 6709.446 | Mortality | LC50 |
| 1912249 | Daphnia carinata | Water Flea | Crustaceans | 6060 | 6776.54046 | Mortality | LC50 |
| 1912249 | Artemia franciscana | San Francisco Brine Shrimp | Crustaceans | 6134 | 6859.290294 | Mortality | LC50 |
| 1912249 | Psetta maxima | Left-Eyed Flounder, Turbot | Fish | 6250 | 6989.00625 | Mortality | NOEC |
| 28553120 | Daphnia magna | Water Flea | Crustaceans | 1700 | 7071.9609 | Mortality | NOEC |
| 57636 | Sida crystallina | Water Flea | Crustaceans | 5000 | 7098.45 | Mortality | NOEC |
| 57636 | Ceriodaphnia dubia | Water Flea | Crustaceans | 5000 | 7098.45 | Mortality | NOEC |
| 1912249 | Channa punctata | Snake-Head Catfish | Fish | 6405.3 | 7162.669077 | Mortality | LC50 |
| 1912249 | Lithobates pipiens | Leopard Frog | Amphibians | 6500 | 7268.5665 | Mortality | NOEC |
| 1912249 | Bufo americanus | American Toad | Amphibians | 6640 | 7425.12024 | Mortality | LC50 |
| 133062 | Diamesa zernyi | Dipteran | Insects | 8106 | 7427.454846 | Mortality | LC50 |
| 1912249 | Oncorhynchus tshawytscha | Chinook Salmon | Fish | 6800 | 7604.0388 | Mortality | LC50 |
| 1912249 | Bufo americanus | American Toad | Amphibians | 6900 | 7715.8629 | Mortality | NOEC |
| 1912249 | Lepomis macrochirus | Bluegill | Fish | 6900 | 7715.8629 | Mortality | LC50 |
| 1912249 | Lithobates pipiens | Leopard Frog | Amphibians | 6970 | 7794.13977 | Mortality | LC50 |
| 57636 | Chironomus tentans | Midge | Insects | 5500 | 7808.295 | Mortality | NOEC |
| 1912249 | Leuciscus idus | Ide, Silver Or Golden Orfe | Fish | 7000 | 7827.687 | Mortality | LC50 |
| 28553120 | Pimephales promelas | Fathead Minnow | Fish | 1900 | 7903.9563 | Mortality | NOEC |
| 1912249 | Poecilia reticulata | Guppy | Fish | 7100 | 7939.5111 | Mortality | LC50 |
| 57636 | Chironomus tentans | Midge | Insects | 5600 | 7950.264 | Mortality | NOEC |
| 57636 | Chironomus tentans | Midge | Insects | 5600 | 7950.264 | Mortality | NOEC |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 7200 | 8051.3352 | Mortality | LC50 |
| 1912249 | Carassius carassius | Crucian Carp | Fish | 7600 | 8498.6316 | Mortality | LC50 |
| 1912249 | Pacifastacus leniusculus | Signal Crayfish | Crustaceans | 7790 | 8711.09739 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 7800 | 8722.2798 | Mortality | LC50 |
| 57636 | Chironomus tentans | Midge | Insects | 6200 | 8802.078 | Mortality | LOEC |
| 57636 | Chironomus tentans | Midge | Insects | 6200 | 8802.078 | Mortality | LOEC |
| 1912249 | Perca sp. | Perch | Fish | 8000 | 8945.928 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 8000 | 8945.928 | Mortality | LOEC |
| 1806264 | Lithobates pipiens | Leopard Frog | Amphibians | 57.771868 | 9151.098554 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 10000 | 9162.91 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 10000 | 9162.91 | Mortality | LC50 |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 10000 | 9162.91 | Mortality | LC50 |
| 13311847 | Oryzias latipes | Japanese Medaka | Fish | 9250 | 9250 | Mortality | NOEC |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 8627.428 | 9647.543714 | Mortality | LOEC |
| 28553120 | Bufo woodhousei ssp. fowleri | Fowler's Toad | Amphibians | 2351 | 9780.105927 | Mortality | LC50 |
| 1912249 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 9200 | 10287.8172 | Mortality | LOEC |
| 56531 | Danio rerio | Zebra Danio | Fish | 1610.1336 | 10312.26648 | Mortality | LOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 9850 | 11014.67385 | Mortality | LC50 |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 10000 | 11182.41 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 10000 | 11182.41 | Mortality | NOEC |
| 1912249 | Carassius carassius | Crucian Carp | Fish | 10000 | 11182.41 | Mortality | LC50 |
| 1912249 | Carassius carassius | Crucian Carp | Fish | 10000 | 11182.41 | Mortality | LC50 |
| 1912249 | Carassius carassius | Crucian Carp | Fish | 10000 | 11182.41 | Mortality | LC50 |
| 1912249 | Carassius carassius | Crucian Carp | Fish | 10000 | 11182.41 | Mortality | LC50 |
| 1912249 | Leiostomus xanthurus | Spot | Fish | 10000 | 11182.41 | Mortality | NOEC |
| 1912249 | Leiostomus xanthurus | Spot | Fish | 10000 | 11182.41 | Mortality | NOEC |
| 1912249 | Leiostomus xanthurus | Spot | Fish | 10000 | 11182.41 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 10000 | 11182.41 | Mortality | NOEC |
| 1912249 | Azolla caroliniana | Water Fern | Plants | 10000 | 11182.41 | Mortality | LC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 10000 | 11182.41 | Mortality | EC50 |
| 1912249 | Cerastoderma edule | Cockle | Molluscs | 10000 | 11182.41 | Mortality | LC50 |
| 1912249 | Cerastoderma edule | Cockle | Molluscs | 10000 | 11182.41 | Mortality | LC50 |
| 1912249 | Physella acuta | European Physa | Molluscs | 10000 | 11182.41 | Mortality | NOEC |
| 1912249 | Planorbella trivolvis | Snail, Marsh Rams-Horn | Molluscs | 10000 | 11182.41 | Mortality | NOEC |
| 1912249 | Stagnicola elodes | Marsh Pond Snail | Molluscs | 10000 | 11182.41 | Mortality | NOEC |
| 1912249 | Artemia franciscana | San Francisco Brine Shrimp | Crustaceans | 10122 | 11318.8354 | Mortality | LC50 |
| 1912249 | Biomphalaria alexandrina | Snail | Molluscs | 10162 | 11363.56504 | Mortality | LC50 |
| 1912249 | Carassius auratus | Goldfish | Fish | 10594 | 11846.64515 | Mortality | LC50 |
| 50328 | Gammarus duebeni | Scud | Crustaceans | 37100 | 12023.0341 | Mortality | LC50 |
| 1912249 | Pacifastacus leniusculus | Signal Crayfish | Crustaceans | 10840 | 12121.73244 | Mortality | LC50 |
| 1912249 | Poecilia reticulata | Guppy | Fish | 11510 | 12870.95391 | Mortality | LC50 |
| 57636 | Hyalella azteca | Scud | Crustaceans | 9100 | 12919.179 | Mortality | NOEC |
| 1912249 | Poecilia reticulata | Guppy | Fish | 11740 | 13128.14934 | Mortality | LC50 |
| 28553120 | Daphnia magna | Water Flea | Crustaceans | 3200 | 13311.9264 | Mortality | NOEC |
| 1912249 | Ceriodaphnia reticulata | Water Flea | Crustaceans | 12000 | 13418.892 | Mortality | LOEC |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 12000 | 13418.892 | Mortality | LOEC |
| 1912249 | Psetta maxima | Left-Eyed Flounder, Turbot | Fish | 12500 | 13978.0125 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 12600 | 14089.8366 | Mortality | LC50 |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 13170 | 14727.23397 | Mortality | EC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 13300 | 14872.6053 | Mortality | LC50 |
| 28553120 | Danio rerio | Zebra Danio | Fish | 3788.484755 | 15760.00945 | Mortality | LC50 |
| 50328 | Crassostrea gigas | Pacific Oyster | Molluscs | 50463.06 | 16353.61432 | Mortality | NOEC |
| 50328 | Crassostrea gigas | Pacific Oyster | Molluscs | 50463.06 | 16353.61432 | Mortality | NOEC |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 15000 | 16773.615 | Mortality | LOEC |
| 56531 | Danio rerio | Zebra Danio | Fish | 2683.556 | 17187.11081 | Mortality | NOEC |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 15800 | 17668.2078 | Mortality | EC50 |
| 1912249 | Americamysis bahia | Opossum Shrimp | Crustaceans | 17000 | 19010.097 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 18000 | 20128.338 | Mortality | LOEC |
| 1912249 | Bufo americanus | American Toad | Amphibians | 19000 | 21246.579 | Mortality | NOEC |
| 28553120 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 5200 | 21631.8804 | Mortality | NOEC |
| 1912249 | Uca pugilator | Fiddler Crab | Crustaceans | 19785 | 22124.39819 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 20000 | 22364.82 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 20000 | 22364.82 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 20000 | 22364.82 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 20000 | 22364.82 | Mortality | NOEC |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 20000 | 22364.82 | Mortality | NOEC |
| 1912249 | Elodea canadensis | Waterweed | Plants | 20000 | 22364.82 | Mortality | NOEC |
| 1912249 | Pacifastacus leniusculus | Signal Crayfish | Crustaceans | 21450 | 23986.26945 | Mortality | LC50 |
| 1912249 | Daphnia carinata | Water Flea | Crustaceans | 22400 | 25048.5984 | Mortality | EC50 |
| 1912249 | Daphnia carinata | Water Flea | Crustaceans | 23100 | 25831.3671 | Mortality | EC50 |
| 1912249 | Utterbackia imbecillis | Paper Pondshell | Molluscs | 24130 | 26983.15533 | Mortality | LC50 |
| 1912249 | Daphnia carinata | Water Flea | Crustaceans | 24600 | 27508.7286 | Mortality | EC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 25000 | 27956.025 | Mortality | LC50 |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 25000 | 27956.025 | Mortality | LC50 |
| 1912249 | Daphnia carinata | Water Flea | Crustaceans | 25300 | 28291.4973 | Mortality | EC50 |
| 1912249 | Daphnia carinata | Water Flea | Crustaceans | 26700 | 29857.0347 | Mortality | EC50 |
| 28553120 | Lepomis microlophus | Redear Sunfish | Fish | 7185 | 29889.43475 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 27068.55535 | 30269.1684 | Mortality | LC50 |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 27600 | 30863.4516 | Mortality | LOEC |
| 13311847 | Oryzias latipes | Japanese Medaka | Fish | 32000 | 32000 | Mortality | NOEC |
| 1912249 | Acartia tonsa | Calanoid Copepod | Crustaceans | 29000 | 32428.989 | Mortality | NOEC |
| 50328 | Crassostrea gigas | Pacific Oyster | Molluscs | 100926.12 | 32707.22863 | Mortality | LOEC |
| 50328 | Crassostrea gigas | Pacific Oyster | Molluscs | 100926.12 | 32707.22863 | Mortality | LOEC |
| 1912249 | Ceriodaphnia reticulata | Water Flea | Crustaceans | 30000 | 33547.23 | Mortality | NOEC |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 30000 | 33547.23 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 30000 | 33547.23 | Mortality | LOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 30000 | 33547.23 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 30000 | 33547.23 | Mortality | NOEC |
| 1912249 | Lampsilis fasciola | Wavy-Rayed Lampmussel | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Lampsilis fasciola | Wavy-Rayed Lampmussel | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Elliptio complanata | Mussel, Eastern Elliptio | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Lampsilis fasciola | Wavy-Rayed Lampmussel | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Villosa constricta | Notched Rainbow | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Villosa constricta | Notched Rainbow | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Villosa delumbis | Eastern Creekshell | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Villosa delumbis | Eastern Creekshell | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 30000 | 33547.23 | Mortality | EC50 |
| 1912249 | Pseudosida ramosa | Water Flea | Crustaceans | 32000 | 35783.712 | Mortality | NOEC |
| 1912249 | Cyprinodon variegatus | Sheepshead Minnow | Fish | 32000 | 35783.712 | Mortality | NOEC |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 35000 | 39138.435 | Mortality | LOEC |
| 1912249 | Mytilus edulis | Common Bay Mussel,Blue Mussel | Molluscs | 35830 | 40066.57503 | Mortality | NOEC |
| 1912249 | Daphnia pulex | Water Flea | Crustaceans | 36500 | 40815.7965 | Mortality | EC50 |
| 1912249 | Lampsilis siliquoidea | Lamp-Mussel | Molluscs | 37500 | 41934.0375 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 40000 | 44729.64 | Mortality | LOEC |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 41000 | 45847.881 | Mortality | NOEC |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 42000 | 46966.122 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 42000 | 46966.122 | Mortality | LOEC |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 43000 | 48084.363 | Mortality | LOEC |
| 1912249 | Ictalurus punctatus | Channel Catfish | Fish | 43000 | 48084.363 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 44000 | 49202.604 | Mortality | LOEC |
| 1912249 | Daphnia pulex | Water Flea | Crustaceans | 46500 | 51998.2065 | Mortality | EC50 |
| 1912249 | Lithobates catesbeianus | American Bullfrog | Amphibians | 48000 | 53675.568 | Mortality | LC50 |
| 1912249 | Lithobates catesbeianus | American Bullfrog | Amphibians | 48000 | 53675.568 | Mortality | LC50 |
| 1912249 | Lithobates catesbeianus | American Bullfrog | Amphibians | 48000 | 53675.568 | Mortality | LC50 |
| 1912249 | Lithobates catesbeianus | American Bullfrog | Amphibians | 48000 | 53675.568 | Mortality | LC50 |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 48000 | 53675.568 | Mortality | NOEC |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 50000 | 55912.05 | Mortality | LOEC |
| 1912249 | Lithobates pipiens | Leopard Frog | Amphibians | 51000 | 57030.291 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 60000 | 67094.46 | Mortality | LOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 60000 | 67094.46 | Mortality | LOEC |
| 1912249 | Pimephales promelas | Fathead Minnow | Fish | 67000 | 74922.147 | Mortality | NOEC |
| 1912249 | Artemia franciscana | San Francisco Brine Shrimp | Crustaceans | 68331 | 76410.52577 | Mortality | LC50 |
| 1912249 | Danio rerio | Zebra Danio | Fish | 80000 | 89459.28 | Mortality | LOEC |
| 133062 | Cancer magister | Dungeness Or Edible Crab | Crustaceans | 100000 | 91629.1 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 90000 | 100641.69 | Mortality | NOEC |
| 1912249 | Neopanope texana | Mud Crab | Crustaceans | 100000 | 111824.1 | Mortality | LC50 |
| 1912249 | Neopanope texana | Mud Crab | Crustaceans | 100000 | 111824.1 | Mortality | LC50 |
| 1912249 | Neopanope texana | Mud Crab | Crustaceans | 100000 | 111824.1 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 100000 | 111824.1 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 100000 | 111824.1 | Mortality | LC50 |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 100000 | 111824.1 | Mortality | NOEC |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 100000 | 111824.1 | Mortality | NOEC |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 100000 | 111824.1 | Mortality | NOEC |
| 1912249 | Daphnia magna | Water Flea | Crustaceans | 100000 | 111824.1 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 100000 | 111824.1 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 100000 | 111824.1 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 100000 | 111824.1 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 100000 | 111824.1 | Mortality | NOEC |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 104000 | 116297.064 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 118000 | 131952.438 | Mortality | NOEC |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 122500 | 136984.5225 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 150000 | 167736.15 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 159600 | 178471.2636 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 169000 | 188982.729 | Mortality | NOEC |
| 1912249 | Eurytemora affinis | Calanoid Copepod | Crustaceans | 175000 | 195692.175 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 176800 | 197705.0088 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 180000 | 201283.38 | Mortality | NOEC |
| 1912249 | Rhinella arenarum | Argentine Toad | Amphibians | 189000 | 211347.549 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 200000 | 223648.2 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 200000 | 223648.2 | Mortality | NOEC |
| 1912249 | Arrenurus sp. | Water Mite | Insects | 200000 | 223648.2 | Mortality | NOEC |
| 1912249 | Artemia franciscana | San Francisco Brine Shrimp | Crustaceans | 236061 | 263973.0887 | Mortality | LC50 |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 250000 | 279560.25 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 274200 | 306621.6822 | Mortality | NOEC |
| 1912249 | Chironomus tentans | Midge | Insects | 280000 | 313107.48 | Mortality | NOEC |
| 1912249 | Danio rerio | Zebra Danio | Fish | 300000 | 335472.3 | Mortality | NOEC |
| 1912249 | Marisa cornuarietis | Snail | Molluscs | 300000 | 335472.3 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 308000 | 344418.228 | Mortality | NOEC |
| 28553120 | Xenopus laevis | African Clawed Frog | Amphibians | 85930 | 357466.8236 | Mortality | LC50 |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 335000 | 374610.735 | Mortality | NOEC |
| 1912249 | Xenopus laevis | African Clawed Frog | Amphibians | 338600 | 378636.4026 | Mortality | NOEC |
| 1912249 | Utterbackia imbecillis | Paper Pondshell | Molluscs | 451200 | 504550.3392 | Mortality | NOEC |
| 1912249 | Oncorhynchus mykiss | Rainbow Trout | Fish | 1078428.5 | 1205942.964 | Mortality | LC50 |
| 1912249 | Hyalella azteca | Scud | Crustaceans | 1201000 | 1343007.441 | Mortality | NOEC |
| 133062 | Procambarus clarkii | Red Swamp Crayfish | Crustaceans | 1563100 | 1432254.462 | Mortality | LC50 |
| 28553120 | Xenopus laevis | African Clawed Frog | Amphibians | 500000 | 2079988.5 | Mortality | LOEC |
| 28553120 | Xenopus laevis | African Clawed Frog | Amphibians | 1000000 | 4159977 | Mortality | NOEC |
| 1912249 | Uca pugnax | Fiddler Crab | Crustaceans | 10000000 | 11182410 | Mortality | NOEC |
| 28553120 | Xenopus laevis | African Clawed Frog | Amphibians | 5000000 | 20799885 | Mortality | NOEC |
| 28553120 | Xenopus laevis | African Clawed Frog | Amphibians | 5000000 | 20799885 | Mortality | NOEC |
| 28553120 | Xenopus laevis | African Clawed Frog | Amphibians | 5000000 | 20799885 | Mortality | NOEC |
| 1912249 | Uca pugnax | Fiddler Crab | Crustaceans | 20000000 | 22364820 | Mortality | LOEC |
| 1912249 | Anopheles arabiensis | Mosquito | Insects | 21568570 | 24118859.29 | Mortality | NOEC |
| 1912249 | Anopheles arabiensis | Mosquito | Insects | 21568570 | 24118859.29 | Mortality | NOEC |
| 1912249 | Anopheles arabiensis | Mosquito | Insects | 21568570 | 24118859.29 | Mortality | NOEC |
| 1912249 | Anopheles arabiensis | Mosquito | Insects | 21568570 | 24118859.29 | Mortality | NOEC |
| 1912249 | Sesarma cinereum | Driftline Crab | Crustaceans | 100000000 | 111824100 | Mortality | NOEC |

Table S4 Anti-AR activity of raw water from six cities in China (BEQFLU)

| Sample place | BEQFLU （μg/L） | SD |
| --- | --- | --- |
| Jinan | 0.609702871 | 0.18615695 |
| Wuhan | 0.546106996 | 0.045516795 |
| Shanghai | 1.592943917 | 0.261324537 |
| Zhuhai | 0 |  |
| Haerbin | 0 |  |
| Chongqing | 0 |  |

## Fig. S11 Species sensitivity distribution based on BEQFLU of compounds for anti-AR activity