Supplementary File 1: Supplementary materials to this study.

Eq S1: The forest carbon sink over a given period t can be expressed in a stock-change framework as follows[1]:

Csink=∆Cbio+∆Cres+∆Csoil #S1

where Csink denotes the stock-change-based forest carbon sink during period t, ΔCbio is the change in living biomass carbon pool, ΔCres is the change in plant residue (litter and coarse woody debris) carbon pool, and ΔCsoil is the change in soil organic carbon pool over the same period. This formulation quantifies a stock-based estimate of carbon gain and is conceptually distinct from flux-based indicators such as net ecosystem productivity (NEP) and net biome productivity (NBP) used in carbon cycle studies.

Eq S2-4: The carbon stock of trees is commonly estimated using allometric equations that relate aboveground biomass to easily measured tree attributes:

W=aDb #S2

W=aD2Hb#S3

Cstock=W×Ccontent #S4

where W is tree biomass (kg), D is diameter at breast height (cm), H is tree height (m), a and b are species-specific parameters (scaled), Cstock is carbon stock (kg C), and Ccontent is carbon content (g C kg-1)[2]. These equations provide a practical way to convert forest inventory measurements into biomass, which can then be multiplied by an assumed carbon fraction to obtain tree carbon stocks.

Eq S5: To reduce uncertainty associated with constant biomass expansion factors, a continuous biomass expansion factor (BEF) approach can be used to express BEF as a function of stand volume[3]:

BEF=a+bx #S5

in this equation, BEF is the biomass expansion factor (dimensionless), x is the stand volume (m3 ha-1), a and b are constants for a specific forest type. Continuous BEF functions allow woody biomass to be estimated more realistically across a range of stand development stages and site conditions, improving stock-change-based carbon sink estimates.

Eq S6-7: For shrubs and small woody plants with tree-like architecture, aboveground biomass can be estimated using shrub-specific allometric equations[4]:

W=a×C×Hb #S6

Cstock=W×Ccontent×4412 #S7

where W is the aboveground biomass (kg), C is crown width (m), H is the plant height (m), and the coefficients (e.g. a, b) are empirically derived parameters. Cstock is the aboveground carbon stock of the shrub (kg), Ccontent is the carbon content, and 4412 is the ratio between CO2 and carbon. Shrub carbon storage is then obtained by multiplying biomass by a carbon concentration factor and, where needed, by a conversion factor between CO2 and elemental carbon. These equations relate shrub biomass to structural attributes such as height and crown width, and are calibrated for particular species or vegetation types.

Eq S8: Carbon storage can be derived directly from destructive sampling in representative plots by measuring the dry mass of plant components and their carbon concentration[5]. Plot-level biomass and carbon estimates are then scaled to the stand or landscape level using appropriate expansion factors. Litter carbon density can be calculated from measured litter mass and its carbon concentration:

Clitter=Bl×θl #S8

where Clitter is litter carbon density (Mg C ha-1), Bl is litter biomass per unit area (Mg ha-1), and θl is the carbon concentration of litter (g kg-1). This pool contributes to the non-living organic matter component of inventory-based carbon stock assessments.

Eq S9-10: Dead wood carbon stocks are generally estimated by combining information on wood volume, decay-class-specific wood density, and assumed carbon concentration:

Vf=π2×D×10−18×Lns+Lew #S9

CDWfd=Vf×Msd×CC×10−4 #S10

where Vf (m3 ha-1) represents the wood volume of fallen logs. D (mm) is the diameter of fallen logs. Lns (m) and Lew (m) represent the horizontal length of the north-south line and the horizontal length of the east-west line, respectively. CDWfd (kg m-2) denotes the carbon stock of fallen dead wood (logs), which is calculated from the product of log volume, decay-class-specific wood density, and an assumed carbon concentration (commonly 500 g kg-1 regardless of species or decay class). d is the decay class of dead wood. Msd (g cm-3) denotes the wood density classified according to decay classes (0-5) for each species category, and CC (g kg-1) is carbon concentration; the carbon concentration of dead logs was set at 500 g kg-1 regardless of species or decay level. This provides an inventory-based estimate of the dead wood pool on the forest floor. This approach can be applied separately to fallen logs, stumps, and standing dead trees, recognising that wood density and decay status differ among components and over time.

Eq S11-12: Stumps and standing dead trees (snags) can be measured using belt-transect methods, where diameter, height, and decay class are recorded for each individual. Their carbon stocks are then computed using analogous equations to those for fallen logs, with volume estimated from stump height and diameter or snag dimensions:

CDWssd=Vss×Msd×CC×10−6 #S11

Vss=Vs1+…+Vsi+…Vsm+VD1+…VDj+…+VDn2×Lns+Lew #S12

where CDWssd (kg m-2) represents the carbon stock of stumps and snags, derived from estimated volume, appropriate wood density values, and a fixed carbon concentration (e.g. 500 g kg-1)[6]. Vss (cm3 m-2) is the wood volume of stumps and snags. VSi (cm3) denotes volume of the ith stump. m represents the total number of stumps measured at the relevant grid point. VDj (cm3) is the volume of the jth snag. n is the total number of snags measured at the relevant grid point. Msd (g cm-3) denotes the wood density classified according to decay classes (0-5) for each species category. Together with fallen dead wood, this component completes the dead wood pool in stock-based accounting.

Eq S13: Soil organic carbon density is estimated from soil samples collected at defined depth intervals, combined with measurements of bulk density, soil water content, gravel fraction, and carbon concentration[7]:

SOCDj=SOCCj×BDj×Psj×wj×102=SOCCj×SDWjV×wj×102 #S13

in the equations, SOCD denotes soil organic carbon density (Mg C ha-1), SOCC is soil organic carbon concentration (g kg-1), SDW is soil dry weight, v is volume of cutting ring, BD indicates bulk density, while Ps is the volume percentage of gravel, and w is soil layer thickness (cm). Total soil organic carbon storage (SOCS) within a given depth range is obtained by summing SOCD across layers and multiplying by soil area.

Eq S14-15: When turbulence is weak, a portion of CO2 may accumulate in the canopy air space and below the eddy covariance (EC) measurement height, leading to a storage term that must be considered in the computation of net ecosystem exchange (NEE). In such cases, NEE is calculated as the sum of the turbulent flux and the change in storage term:

NEE=FC+FS #S14

where NEE represents the net ecosystem exchange (μmol CO2 m-2 s-1), FC is the turbulent CO2 flux measured by the EC system (μmol CO2 m-2 s-1) , and FS is the storage flux (μmol CO2 m-2 s-1). The latter is often approximated as the change in mean CO2 concentration between two time steps multiplied by the height of the measurement level:

FS=∆c∆t×h #S15

Δc is the change in CO2 concentration (μmol m-3) between two consecutive measurements, Δt is the time interval between two consecutive measurements, and h is the measurement height (m)[8].

Eq S16: For periods with missing or low-quality EC data, gap-filling is typically performed using lookup tables or data-driven relationships between fluxes and meteorological variables. A generic formulation can be written as:

Ffilled=LUTTair,Rg,VPD #S16

- where Ffilled represents the gap-filled CO2 flux (μmol CO2 m-2 s-1), LUT is look-up table function, Tair is air temperature (℃), Rg (W m-2) is global radiation, VPD is vapor pressure deficit (kPa). This formulation is conceptually consistent with the Marginal Distribution Sampling (MDS) approach widely adopted in FLUXNET.

- References

- [1] Pan YD, Birdsey RA, Phillips OL, Houghton RA, Fang JY, et al. 2024. The enduring world forest carbon sink. Nature 631:563-569.

- [2] Fang JY, Liu GH, Zhu B, Wang XK, Liu SH. 2007. Carbon budgets of three temperate forest ecosystems in Dongling Mt., Beijing, China. Science in China Series D: Earth Sciences 50:92-101.

[3] Fang JY, Chen AP, Peng CH, Zhao SQ, Ci L. 2001. Changes in Forest Biomass Carbon Storage in China Between 1949 and 1998. Science 292:2320-2322.

[4] Zhang XY, 2019. Quick aboveground carbon stock estimation of densely planted shrubs by using point cloud derived from unmanned aerial vehicle. Remote Sensing 11:2914.

[5] Tang XL, Zhao X, Bai YF, Tang ZY, Wang WT, et al. 2018. Carbon pools in China's terrestrial ecosystems: New estimates based on an intensive field survey. Proceedings of the National Academy of Sciences of the United States of America 115:4021-4026.

[6] Kawanishi A, Sakai Y, Ishizuka S, Hashimoto S, Komatsu M, et al. 2024. Increased deadwood carbon stocks through planted forestry practices: insights from a Forest Inventory Survey in Japan. Carbon Management 15:2315087.

[7] Hong SB, Ding JZ, Kan F, Xu H, Chen SY, et al. 2023. Asymmetry of carbon sequestrations by plant and soil after forestation regulated by soil nitrogen. Nature Communications 14:3196.

[8] Zhu N, Wang JN, Luo DL, Wang XF, Shen C, et al. 2024. Using eddy covariance observations to determine the carbon sequestration characteristics of subalpine forests in the Qinghai-Tibet Plateau. Biogeosciences 21:3509-3522.

[9] Zhu XJ, Fan RX, Chen Z, Wang QF, Yu GR. 2022. Eddy covariance-based differences in net ecosystem productivity values and spatial patterns between naturally regenerating forests and planted forests in China. Scientific Reports 12:20556.

[10] Familusi I, Gebremedhin M, Gyawali B, Chiluwal A, Brotzge J. 2025. A Deciduous Forest’s CO2 Exchange Within the Mixed-Humid Climate of Kentucky, USA. Forests 16:562.

[11] Pulliainen J, Aurela M, Aalto T, Böttcher K, Cohen J, et al. 2024. Increase in gross primary production of boreal forests balanced out by increase in ecosystem respiration. Remote Sensing of Environment 313:114376.

[12] Pilli R, Alkama R, Cescatti A, Kurz WA, Grassi G. 2022. The European forest carbon budget under future climate conditions and current management practices. Biogeosciences 19:3263-3284.

[13] Yan Y, Wang X, Wohlfahrt G, Huang N. 2025. Seasonal and interannual variations of soil heterotrophic respiration and autotrophic respiration in subtropical forests of southeast China: independent process-based models. Geoscience Letters 12:27.

[14] Evans LR, Pierson D, Lajtha K. 2020. Dissolved organic carbon production and flux under long-term litter manipulations in a Pacific Northwest old-growth forest. Biogeochemistry 149:75-86.

[15] Tan S, Yue K, Peñuelas J, Svenning JC, Meerbeek KV, et al. 2025. Forest streams may discharge one tenth of the annual total global forest carbon sink through dissolved organic carbon. The Innovation Geoscience 3:100148.

[16] Petersen RC, Holst T, Wu C, Krejci R, Chan JK, et al. 2025. BVOC and speciated monoterpene concentrations and fluxes at a Scandinavian boreal forest. Atmospheric Chemistry and Physics 25:17205-17236.