Text S1.

Detailed calculation of NH3 flux for primary monitoring techniques

- Closed and vented chambers

The closed chamber method utilizes a sealed enclosure with an acidic trap, and the vented chamber uses acid-soaked sponges to capture NH3[1].

NH3 emission flux (mg N m−2 h−1) is calculated by the following formula:

F=Mf−MbA×t

where Mf (mg N) is NH4+-N content in the acidic solution (closed chamber) or the lower sponge (vented chamber) for N-applied plots; Mb (mg N) is the NH4+-N content in sponge before it was placed in field; A (m2) is the inner area of the chamber. t (h) is the duration the sponge is placed in the field.

- Dräger-Tubes

Ambient air is drawn through four chambers by a hand pump. The air is enriched with NH3 emitted from the soil surface. The air is then led through Teflon tubing to an NH3 sensitive Dräger gas analysis detector tube which immediately displays the NH3 concentration. The NH3 fluxes (mg N m−2 h−1) were calculated according to the following formula:

F=V×C×10−6×ρNH3×UN×UF×UZ

where V is air volume sucked through the chambers; C is the value of NH3 vol. concentration (ppm); ρNH3 is temperature-dependent density of NH3 (mg L−1); UN is molecular weight conversion factor of NH3 to N; UF is surface area conversion factor (m−2); UZ is time conversion factor (h−1). Since the air exchange rate in the chamber is very low (1 vol. min−1) , the actual NH3 emission is underestimated, and the calibration should be conducted as described by Pacholski et al.[2].

- Dynamic chambers

Air is drawn through the chamber at a known rate by a vacuum pump, and the exhausted air is passed through acid solution to trap NH3. The concentration of NH4+‒N in the absorption liquid is then measured to calculate the NH3 emission fluxes. The NH3 fluxes (kg N ha−1 d−1) were calculated using the following equation:

F=C×V×10−2π×r2×(t/24)

where C is the NH4+-N concentration in the absorption solution (mg N L−1); V is the volume of the absorption solution after measurement (L); t is the duration of collection (h); and r is the chamber radius (m).

- Wind tunnel

Wind speed inside the tunnel is adjusted to match the external environment through an automatic fan system, ensuring that factors such as temperature, humidity, and light inside and outside the tunnel remain relatively consistent. Compared to the static and dynamic chamber methods, it enhances the microclimatic conditions of the experimental area while retaining the chamber method’s advantage of suitability for small-scale, multi-treatment comparative experimental studies[3-7]. A half-cylinder or rectangular tunnel is placed over the soil surface, and a fan forces air through it at controlled speeds to simulate specific aerodynamic conditions. Driven by the fan, the air flow stably flows through the wind tunnel. NH3 emission flux is obtained by measuring the mass difference of NH3 before and after the wind tunnel intake. The NH3 emission fluxes (g N m−2 min−1) were calculated using the following equations [8]:

F=C×qA

where C is the NH3 concentration (g L−1), q is the volumetric air flow rate (L min−1), and A is the area of the soil surface within the tunnel (m2).

C=(C0−Ci)×P×MR×T

P is the air pressure (1 atm), R is the gas constant (0.08206 L atm K−1 mol−1), T is the temperature (K), Co is the outlet concentration (ppb), Ci is the inlet concentration (ppb), and M is the molar mass of NH3 (17.03 g mol−1).

This method has some drawbacks, for instance, the equipment is complex and expensive, the operation is cumbersome, and importantly, the wind tunnel method cannot simulate calm wind conditions or precipitation. When wind speeds fall below 0.3 m s−1, measurement errors tend to increase.

- Integrated horizontal flux (IHF)

The vertical NH3 flux in a region equals to the integration of horizontal NH3 fluxes across a vertical cross-section on a rod[9,10]. This method is suitable only for measuring NH3 emissions following N fertilization in cropland. At any given height, the horizontal NH3 flux is the product of wind speed and NH3 concentration. A gaseous NH3 absorption device and an anemometer were placed at different heights on a mast positioned at the center of the circular area, ensuring that the NH3 concentration at the highest point approximates the background concentration.

Vertical flux (F, g m−2 s−1) is calculated by integrating the net horizontal fluxes over the height profile (from 0 to Z, m) and dividing by the fetch distance or plot radius (x, m) to obtain the NH3 flux, using the following equation[11-13]:

F=1x0Z(uc−ucb)dZ

where u (m s−1) is the wind speed, and c (g m−3) is NH3 concentration downwind, and cb is the NH3 concentration at the upwind edge of the treated area.

The invention of the upwind sampler by Leuning has simplified and made the operation of the mass balance method more feasible. This sampler can directly measure the horizontal flux of NH3 at a specific height without the need to separately measure wind speed and NH3 concentration[14]. According to Leuning et al. (1985), ȗĉ is derived from:

uc=MAt

where M is the amount of NH3-N collected (g) by the Leuning sampler during the sampling period t (s), and A is the effective cross-sectional area of the sampler (m2), determined in wind-tunnel calibrations.

- Aerodynamic gradient method (AGM)

Over an extensive, uniform surface, measurements of the vertical profiles of wind speed, temperature, and NH3 concentration can be used to estimate fluxes between the atmosphere and the ground. A negative flux value denotes the absorption of atmospheric NH3 by the underlying soil–canopy system[15].

The principle of the AGM is that the NH3 flux is calculated from vertical concentration profiles of NH3, coupled with turbulence information from either combined wind and temperature profiles or from ultrasonic anemometers, which provide direct estimates of friction velocity and sensible heat flux[16]. The NH3 flux (F) was calculated using the following equation[17,18]:

F=−u∗k∂x∂ln(z−d)−ψH(z−d)/L

where u∗is the friction velocity, k is the von Kármán constant (0.41), χ is NH3 concentration, z is height above the ground, and ψH is the integrated stability function, which changes with height (z) above the zero-plane displacement (d) and atmospheric stability, parameterized by the Monin-Obukhov length (L).

Another similar gradient method was adopted by Hayashi et al. (2009). The NH3 flux was calculated by multiplying the difference in NH3 concentrations between two heights by the diffusion velocity (D, m s−1)[19], which is calculated as:

D=u∗kζ1ζ2ϕg(ζ)ζ−1dζ−1

where u*, k, and ϕg(ζ) are von Karman’s constant (0.4), the friction velocity (m s−1), and the correction factor as a function of the Monin–Obukhov stability parameter (ζ), respectively. ζ is defined as ζ= (z-d)/L, where z, d, and L are the target height (m), zero-plane displacement height, and the Monin–Obukhov length (m), respectively. ζ1 and ζ2 are the ζ values at heights of z1 and z2 respectively. ϕg was approximated by the dimensionless gradient of the potential temperature (ϕh). z and ϕg(ζ) were calculated using u* and L from the sonic anemometer-thermometer. The mean value of D during the measurement interval of NH3 was used for NH3 flux calculation.

- Relaxed eddy accumulation method (REA)

The REA method can also be used to investigate the net bidirectional exchange of NH3 (emission/absorption) above the canopy under field conditions[20]. The REA method is based on the principles of the eddy covariance technique and has the advantage of not requiring high time-resolution concentration measurements, such as those obtained with denuder or filter-pack sampling systems[20]. Additionally, a 3D sonic anemometer is necessary. The NH3 flux is derived from the difference in NH3 concentration between updrafts and downdrafts as follows:

F=βσw(C+−C−)

where β is an empirical coefficient that describes the relative contributions of turbulent updrafts and downdrafts to the flux, with a theoretical value of 0.627[21], σw is the standard deviation of the vertical wind velocity, and C+ and C- are the mean concentrations of NH3 in the updraft and downdraft, respectively.

- Eddy covariance (EC)

This method requires fast-response, and highly sensitive NH3 analyzers, such as a solar-powered open-path NH3 analyzer, and a 3D sonic anemometer. Currently, EddyPro software is used to calculate EC fluxes following widely accepted methodologies[22].

The following equation is used to calculate the NH3 flux:

F=Aw'ρA'+BμρAρdw'ρv'+C(1+μρvρd)ρATaw'Ta'

According to Wang et al. (2021), ρA is the NH3 density; ρd is the dry air density; μ is the molar mass ratio of dry air to water vapor; ρv is the water vapor density; w′ρ′ is the water vapor flux; Ta is the air temperature; and w′Ta′ is the heat flux. The variables A, B, and C are dimensionless parameters and account for spectroscopic effects, varying with ambient temperature, pressure, and water vapor content.

- Backward Lagrangian stochastic (bLS) model

The NH3 emission flux (Q, μg m−2 s−1) can then be calculated using the following equation :

Q=(c−cb)(c/Q)sim

where c (μg m−3) represents NH3 concentration at the monitoring location, while cb denotes the background NH3 concentration. NH3 concentration can be measured using devices such as the open-path tunable diode laser absorption spectroscopy (TDLAS)[23], the mini-differential optical absorption spectroscopy (DOAS) system[24], or the open-path Fourier transform infrared (OP-FTIR) spectroscopic concentration sensor[25].

The ratio (c/Q)sim is predicted by modeling the upwind transport trajectories of NH3 from the monitoring site back to its source, based on Monin–Obukhov similarity theory (MOST) where wind flow is determined by atmospheric friction velocity (u*), the Obukhov stability length (L), the surface roughness length (z0) and wind direction (β), which can be obtained by measuring profiles of temperature and wind[26].

Table S1. Comparison of current cropland NH3 emission inventories

| Studies | Year | Global | China | Method | Fertilizer type |
| --- | --- | --- | --- | --- | --- |
| Yan et al.[27] | 1995 |  | 5.77 | Constant EF | FSN, FAW, FBN and FCRa |
| Bouwman et al.[28] | 1995 | 9.0 |  | Constant EF | FSN |
| Bouwman et al.[29] | 2001 | 18.5 |  | Linear regression model | FSN and FAW |
| Riddick et al.[30] | 2000 | 12.0 | 2.6 | Process-based model | FSN |
| Xu et al.[31] | 2000 | 13.6 | 3.3 | DLEM-Bi-NH3 | FSN |
|  | 2010 | 16.7 | 4.2 | DLEM-Bi-NH3 | FSN |
| IPCC Tier 1 | 2000 | 13.8 |  | Constant EF | FSN, FAW, and FCR |
| Paulot et al.[32] | 2005-2008 | 9.4 | 3.0 | Data-driven model | FSN |
| Vira et al.[33] | 2010-2015 | 11.0 | 2.3 | Process-based model | FSN |
| Zhan et al.[34] | 2000 | 14.4 | 4.5 | Updated data-driven model | FSN, FAW, and FCR |
| Xu et al.[35] | 2018 | 4.3 | 1.4 | Machine learning model | FSN and FAW in rice, maize and wheat fields |
| Zhang et al.[36] | 2020 | 22 |  | CHANS mass-balance model | All N fertilizer input |
| Zhang et al.[37] | 2015 |  | 8.2 | CHANS mass-balance model | FSN, FAW |
| Wang et al.[38] | 2013 |  | 1.7 | Multi-place field measurements | FSN |

aFSN, synthetic N fertilizer; FAW, animal excreta used as fertilizer; FBN, biologically fixed N; FCR, N in crop residue returned to croplands

References

[1] Wang ZH, Liu XJ, Ju XT, Zhang FS, Malhi SS. 2004. Ammonia volatilization loss from surface-broadcast urea: comparison of vented- and closed-chamber methods and loss in winter wheat–summer maize rotation in North China Plain. Communications in Soil Science and Plant Analysis 35(19−20):2917−2939 https://doi.org/10.1081/CSS-200036499

[2] Pacholski A, Cai G, Nieder R, Richter J, Fan X, et al. 2006. Calibration of a simple method for determining ammonia volatilization in the field–comparative measurements in Henan Province, China. Nutrient Cycling in Agroecosystems 74(3):259−273 https://doi.org/10.1007/s10705-006-9003-4

[3] Bouwmeester RJB, Vlek PLG. 1981. Wind-tunnel simulation and assessment of ammonia volatilization from ponded Water1. Agronomy Journal 73(3):546−552 https://doi.org/10.2134/agronj1981.00021962007300030033x

[4] Loubet B, Cellier P, Flura D, Génermont S. 1999. An evaluation of the wind-tunnel technique for estimating ammonia volatilization from land: part 1. analysis and improvement of accuracy. Journal of Agricultural Engineering Research 72(1):71−81 https://doi.org/10.1006/jaer.1998.0348

[5] Rochette P, Angers DA, Chantigny MH, MacDonald JD, Gasser MO, et al. 2009. Reducing ammonia volatilization in a no-till soil by incorporating urea and pig slurry in shallow bands. Nutrient Cycling in Agroecosystems 84(1):71−80 https://doi.org/10.1007/s10705-008-9227-6

[6] Zhang C, Li XQ, Su F, Zhu B, Ju XT. 2016. Effects of different fertilization and measurement methods on ammonia volatilization of summer maize in purple soil. Journal of Agro-Environment Science 35:1194−1201 https://doi.org/10.11654/jaes.2016.06.024

[7] Su F, Ding XQ, Gao ZL, Huang BX, Chen XP, et al. 2007. Ammonia volatilization from nitrogen fertilization of winter wheat-summer maize rotation system in the North China Plain. China Environmental Science 27:409−413

[8] Pedersen JM, Feilberg A, Kamp JN, Hafner S, Nyord T. 2020. Ammonia emission measurement with an online wind tunnel system for evaluation of manure application techniques. Atmospheric Environment 230:117562 https://doi.org/10.1016/j.atmosenv.2020.117562

[9] Denmead OT, Simpson JR, Freney JR. 1977. A direct field measurement of ammonia emission after injection of anhydrous ammonia. Soil Science Society of America Journal 41(5):1001−1004 https://doi.org/10.2136/sssaj1977.03615995004100050039x

[10] Ryden JC, McNeill JE. 1984. Application of the micrometeorological mass balance method to the determination of ammonia loss from a grazed sward. Journal of the Science of Food and Agriculture 35(12):1297−1310 https://doi.org/10.1002/jsfa.2740351206

[11] Cai GX, Fan XH, Yang Z, Zhu ZL. 1998. Gaseous loss of nitrogen from fertilizers applied to wheat on a calcareous soil in North China Plain. Pedosphere 8:45−52

[12] Abalos D, Sanz-Cobena A, Misselbrook T, Vallejo A. 2012. Effectiveness of urease inhibition on the abatement of ammonia, nitrous oxide and nitric oxide emissions in a non-irrigated Mediterranean barley field. Chemosphere 89(3):310−318 https://doi.org/10.1016/j.chemosphere.2012.04.043

[13] Sanz-Cobena A, Misselbrook TH, Arce A, Mingot JI, Diez JA, et al. 2008. An inhibitor of urease activity effectively reduces ammonia emissions from soil treated with urea under Mediterranean conditions. Agriculture, Ecosystems & Environment 126(3−4):243−249 https://doi.org/10.1016/j.agee.2008.02.001

[14] Leuning R, Freney JR, Denmead OT, Simpson JR. 1985. A sampler for measuring atmospheric ammonia flux. Atmospheric Environment (1967) 19(7):1117−1124 https://doi.org/10.1016/0004-6981(85)90196-9

[15] Sutton MA, Flower D, Moncriefe JB. 1993. The exchange of atmospheric ammonia with vegetated surfaces. I: Unfertilized vegetation. Quarterly Journal of the Royal Meteorological Society 119(513):1023−1045 https://doi.org/10.1002/qj.49711951309

[16] Milford C, Theobald MR, Nemitz E, Hargreaves KJ, Horvath L, et al. 2009. Ammonia fluxes in relation to cutting and fertilization of an intensively managed grassland derived from an inter-comparison of gradient measurements. Biogeosciences 6(5):819−834 https://doi.org/10.5194/bg-6-819-2009

[17] Nemitz E, Sutton MA, Wyers GP, Otjes RP, Schjoerring JK, et al. 2000. Surface/atmosphere exchange and chemical interaction of gases and aerosols over oilseed rape. Agricultural and Forest Meteorology 105(4):427−445 https://doi.org/10.1016/S0168-1923(00)00207-0

[18] Sutton MA, Nemitz E, Milford C, Fowler D, Moreno J, et al. 2000. Micrometeorological measurements of net ammonia fluxes over oilseed rape during two vegetation periods. Agricultural and Forest Meteorology 105(4):351−369 https://doi.org/10.1016/S0168-1923(00)00203-3

[19] Hayashi K, Koga N, Yanai Y. 2009. Effects of field-applied composted cattle manure and chemical fertilizer on ammonia and particulate ammonium exchanges at an upland field. Atmospheric Environment 43(35):5702−5707 https://doi.org/10.1016/j.atmosenv.2009.07.043

[20] Xu M, Umehara M, Sase H, Matsuda K. 2022. Ammonia fluxes over an agricultural field in growing and fallow periods using relaxed eddy accumulation. Atmospheric Environment 284:119195 https://doi.org/10.1016/j.atmosenv.2022.119195

[21] Nelson AJ, Koloutsou-Vakakis S, Rood MJ, Myles L, Lehmann C, et al. 2017. Season-long ammonia flux measurements above fertilized corn in central Illinois, USA, using relaxed eddy accumulation. Agricultural and Forest Meteorology 239:202−212 https://doi.org/10.1016/j.agrformet.2017.03.010

[22] Wang K, Kang P, Lu Y, Zheng X, Liu M, et al. 2021. An open-path ammonia analyzer for eddy covariance flux measurement. Agricultural and Forest Meteorology 308:108570 https://doi.org/10.1016/j.agrformet.2021.108570

[23] Yang W, Zhu A, Zhang J, Zhang Y, Chen X, et al. 2013. An inverse dispersion technique for the determination of ammonia emissions from urea-applied farmland. Atmospheric Environment 79:217−224 https://doi.org/10.1016/j.atmosenv.2013.07.005

[24] Voglmeier K, Jocher M, Häni C, Ammann C. 2018. Ammonia emission measurements of an intensively grazed pasture. Biogeosciences 15(14):4593−4608 https://doi.org/10.5194/bg-15-4593-2018

[25] Bai M, Sun J, Denmead OT, Chen D. 2017. Comparing emissions from a cattle pen as measured by two micrometeorological techniques. Environmental Pollution 230:584−588 https://doi.org/10.1016/j.envpol.2017.07.012

[26] Gao Z, Desjardins RL, Flesch TK. 2009. Comparison of a simplified micrometeorological mass difference technique and an inverse dispersion technique for estimating methane emissions from small area sources. Agricultural and Forest Meteorology 149(5):891−898 https://doi.org/10.1016/j.agrformet.2008.11.005

[27] Yan X, Akimoto H, Ohara T. 2003. Estimation of nitrous oxide, nitric oxide and ammonia emissions from croplands in East, Southeast and South Asia. Global Change Biology 9(7):1080−1096 https://doi.org/10.1046/j.1365-2486.2003.00649.x

[28] Bouwman AF, Lee DS, Asman WAH, Dentener FJ, Van Der Hoek KW, et al. 1997. A global high-resolution emission inventory for ammonia. Global Biogeochemical Cycles 11(4):561−587 https://doi.org/10.1029/97gb02266

[29] Bouwman AF, Boumans LJM, Batjes NH. 2002. Estimation of global NH3 volatilization loss from synthetic fertilizers and animal manure applied to arable lands and grasslands. Global Biogeochemical Cycles 16(2):8−1−8−14 https://doi.org/10.1029/2000GB001389

[30] Riddick S, Ward D, Hess P, Mahowald N, Massad R, et al. 2016. Estimate of changes in agricultural terrestrial nitrogen pathways and ammonia emissions from 1850 to present in the Community Earth System Model. Biogeosciences 13(11):3397−3426 https://doi.org/10.5194/bg-13-3397-2016

[31] Xu R, Tian H, Pan S, Prior SA, Feng Y, et al. 2019. Global ammonia emissions from synthetic nitrogen fertilizer applications in agricultural systems: Empirical and process-based estimates and uncertainty. Global Change Biology 25(1):314−326 https://doi.org/10.1111/gcb.14499

[32] Paulot F, Jacob DJ, Pinder RW, Bash JO, Travis K, et al. 2014. Ammonia emissions in the United States, European Union, and China derived by high-resolution inversion of ammonium wet deposition data: Interpretation with a new agricultural emissions inventory (MASAGE_NH3). Journal of Geophysical Research: Atmospheres 119(7):4343−4364 https://doi.org/10.1002/2013JD021130

[33] Vira J, Hess P, Melkonian J, Wieder WR. 2020. An improved mechanistic model for ammonia volatilization in Earth system models: Flow of Agricultural Nitrogen version 2 (FANv2). Geoscientific Model Development 13(9):4459−4490 https://doi.org/10.5194/gmd-13-4459-2020

[34] Zhan X, Adalibieke W, Cui X, Winiwarter W, Reis S, et al. 2021. Improved estimates of ammonia emissions from global croplands. Environmental Science & Technology 55(2):1329−1338 https://doi.org/10.1021/acs.est.0c05149

[35] Xu P, Li G, Zheng Y, Fung JCH, Chen A, et al. 2024. Fertilizer management for global ammonia emission reduction. Nature 626(8000):792−798 https://doi.org/10.1038/s41586-024-07020-z

[36] Zhang X, Sun Y, Gao Y, Wang C, Liang X, et al. 2026. Halving global ammonia emissions with cost-effective measures. Nature Sustainability 9(2):247−259 https://doi.org/10.1038/s41893-025-01723-5

[37] Zhang X, Wu Y, Liu X, Reis S, Jin J, et al. 2017. Ammonia emissions may be substantially underestimated in China. Environmental Science & Technology 51(21):12089−12096 https://doi.org/10.1021/acs.est.7b02171

[38] Wang H, Zhang D, Zhang Y, Zhai L, Yin B, et al. 2018. Ammonia emissions from paddy fields are underestimated in China. Environmental Pollution 235:482−488 https://doi.org/10.1016/j.envpol.2017.12.103