Table S1 Summary of typical pollutants in coastal-marine interfaces (CMIs) of China: ecological risks and human exposure pathways.

| Pollutant category | Typical pollutants | Key ecological risks | Human exposure pathways |
| --- | --- | --- | --- |
| Heavy Metals | Cd, Cr, As, Pb | • Benthic organism bioaccumulation and biomagnification along food chains • Inhibition of microbial metabolic activity (e.g., denitrification, sulfate reduction) • Oxidative stress and DNA damage in aquatic organisms | • Consumption of contaminated shellfish, finfish, and sea salt • Drinking of polluted coastal groundwater • Dermal contact during coastal recreation (minor pathway) |
|  | Cu, Zn, Ni | • Algal growth inhibition and community structure alteration • Toxicity to early life stages of fish and invertebrates • Bioaccumulation in aquaculture species (e.g., shrimp, scallops) • Disruption of photosynthetic processes in macroalgae | • Consumption of aquaculture products (shrimp, scallops, fish) • Drinking of contaminated coastal groundwater • Ingestion of sea salt with elevated metal levels |
| Nutrients (N, P) | NO₃⁻-N, NH₄⁺-N, PO₄³⁻-P, TN, TP | • Harmful algal blooms (HABs, e.g., dinoflagellates, diatoms) and eutrophication • Hypoxia/anoxia in bottom waters, leading to benthic habitat degradation • Shifts in phytoplankton community toward harmful species • Reduction of biodiversity in coastal ecosystems | • Consumption of HAB-contaminated seafood (e.g., paralytic shellfish poisoning) • Drinking of high-nitrate coastal groundwater (methemoglobinemia risk) • Indirect exposure via reduced water quality affecting recreational and economic uses |
| Persistent Organic Pollutants (POPs) | PAHs, PCBs, DDTs, HCHs | • Endocrine disruption in aquatic organisms • Carcinogenic and mutagenic effects on benthic invertebrates and fish • Bioaccumulation and biomagnification along marine food chains • Inhibition of microbial degradation of other pollutants | • Consumption of contaminated fish, shellfish, and sea salt • Inhalation of volatilized POPs from coastal waters (minor pathway) • Dermal contact with contaminated sediment during coastal activities |
| Emerging Contaminants | Microplastics (MPs) | • Physical ingestion and blockage of digestive tracts in aquatic organisms • Leaching of toxic additives (e.g., phthalates, bisphenol A) into surrounding environments • Adsorption and transport of other pollutants as carriers • Alteration of microbial community structure in sediments and water columns | • Consumption of MP-contaminated seafood (ingestion of MPs and associated toxins) • Drinking of MP-containing coastal groundwater • Ingestion of sea salt with MP residues |
|  | Antibiotics | • Induction and proliferation of antibiotic resistance genes (ARGs) in microbial communities • Disruption of microbial nitrogen and carbon cycling processes • Toxicity to aquatic organisms (e.g., growth inhibition, reproductive impairment) • Alteration of benthic and planktonic community structures | • Consumption of antibiotic-contaminated aquaculture products (fish, shrimp, shellfish) • Drinking of antibiotic-containing coastal groundwater • Indirect exposure via ARG dissemination in environmental microbes |
|  | PFAS | • Endocrine disruption and reproductive toxicity in aquatic organisms • Carcinogenic effects on benthic invertebrates and fish • Bioaccumulation and biomagnification along marine food chains • Inhibition of immune system function in marine organisms | • Consumption of PFAS-contaminated fish, shellfish, and sea salt • Drinking of PFAS-polluted coastal groundwater • Inhalation of PFAS-containing aerosols from coastal waters (minor pathway) |

Table S2 National standard thresholds of pollutants in coastal groundwater of China.

| Pollutant category | Specific pollutant | Concentration standard threshold | Applicable national standard (GB) | Standard description/notes |
| --- | --- | --- | --- | --- |
| Heavy Metals | Cadmium (Cd) | ≤ 0.005 mg/L (0.005 μg/mL) | GB 5749-2022 | Standards for Drinking Water Quality; applicable to coastal groundwater used for drinking purposes |
|  | Chromium (Cr) | ≤ 0.05 mg/L (0.05 μg/mL) | GB 5749-2022 | Standards for Drinking Water Quality; refers to total chromium |
|  | Arsenic (As) | ≤ 0.01 mg/L (0.01 μg/mL) | GB 5749-2022 | Standards for Drinking Water Quality; applicable to all types of drinking groundwater sources |
|  | Lead (Pb) | ≤ 0.01 mg/L (0.01 μg/mL) | GB 5749-2022 | Standards for Drinking Water Quality; strict limit due to high toxicity and bioaccumulation |
|  | Copper (Cu) | ≤ 1.0 mg/L (1.0 μg/mL) | GB 5749-2022 | Standards for Drinking Water Quality; limit based on sensory and toxicological effects |
|  | Zinc (Zn) | ≤ 1.0 mg/L (1.0 μg/mL) | GB 5749-2022 | Standards for Drinking Water Quality; excessive levels may cause turbidity and taste issues |
|  | Nickel (Ni) | ≤ 0.02 mg/L (0.02 μg/mL) | GB 5749-2022 | Standards for Drinking Water Quality; based on long-term health risk assessment |
| Nutrients | Nitrate-N (NO₃⁻-N) | ≤ 50 mg/L | GB 5749-2022 | Standards for Drinking Water Quality; limit to prevent methemoglobinemia |
|  | Ammonia-N (NH₄⁺-N) | ≤ 0.5 mg/L (as N) | GB 5749-2022 | Standards for Drinking Water Quality; excessive levels cause odor and taste problems |
|  | Total Phosphorus (TP) | ≤ 0.2 mg/L (for Class Ⅲ groundwater) | GB/T 14848-2017 | Standards for Groundwater Quality; Class Ⅲ is applicable to centralized drinking water sources |
| Emerging Contaminants | Antibiotics (sulfonamides, tetracyclines, quinolones) | ≤ 100 ng/L (for individual antibiotics) | GB 5749-2022 | Standards for Drinking Water Quality; applies to common antibiotics in environmental media |
|  | Microplastics (MPs) | No unified national standard threshold | - | Proposed preliminary risk thresholds (10 items/L) are for reference only |
| Persistent Organic Pollutants (POPs) | PAHs (total) | ≤ 0.002 mg/L | GB 5749-2022 | Standards for Drinking Water Quality; sum of 16 priority PAHs specified by the standard |

Table S3 The species, distribution, major sources, and influencing factors of predominant pollutants in some typical coastal interfaces globally.

| Classification | Species | Sites | Samples | Major sources for pollutant | Content | Influencing processes | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Persistent organic pollutant | PBDEs | Lim Chu Kang, Singapore | Sediment | Industrial waste release/ Organohalide additives | 23.58 ± 2.99 ng g-1 | Plastic type/ Plastisphere Microbiota | [1] |
|  |  | Nhat Le Estuaries, central Vietnam | Sediment core | Historical usage/disposal/ Substantial uses/ongoing releases of deca-BDE | 22 ± 4.4 ng g-1 | Runoff water and riverine discharges | [2] |
|  | PCBs | Southeastern Maritime Bays, South Korea | Sediment | Industrial/ship-related activities/ Metallurgical combustion processes | 3,427 pg g-1 | Organic matter/ Particle size | [3] |
|  |  | Shandong Peninsula, China | Sediment | Industrial thermal processes in nearby shipyards | 6.5 ~ 13.6 ng g-1 | Adsorption by organic matter/ Anaerobic microorganisms/ pH/ Salinity | [4] |
|  |  | Jiaozhou Bay, China | Sediment | Shipping/ Atmospheric transport/ Sedimentation | 15.41 ng g-1 | Water flow speed/ Small particle deposition/ Organic matter accumulation | [5] |
|  | PAHs | Bohai Sea, China | Seawater | Petrogenic/petroleum combustion sources | 10.14 ~ 67.03 ng L-1 (June) 3.75 ~ 32.23 ng L-1 (August) | Temperature/ Light intensity/ Rainfall/ Surface runoff | [6] |
|  |  | Qinhuangdao coastal area, China | Atmospheric | Air−sea exchange/ Vehicle/ship emissions in fishing and tourism | 24.3 ~ 276 ng m3 | Temperature/ Wind/ Evaporation from particle phase/ Humidity | [7] |
|  |  | Coastal Deception, Antarctica | Seawater | Snow melting input/ Long-range atmospheric transport / Wet deposition | 2.9 ~ 27.0 ng L-1 | Microbial degradation/ Sorption and sinking to biological pump/ Photodegradation/ Nutrient availability | [8] |
|  |  | Benya Lagoon, Ghana | Sediment | Shipping activities/ Vehicular emissions/ Waste burning | 36 ~ 824 μg kg-1 | Sediment erosion/ Dilution by sea/ Uptake or synthesising by marshes | [9] |
| Nutrients | Phosphorus | Wakasa Bay, Japan | Surface water | Groundwater discharge | 0.4 ~ 1.7 μmol L-1 | Stratification and vertical mixing/ Topographic gradient | [10] |
|  |  |  | Groundwater | Upstream input | 0.5 ~ 8.0 μmol L-1 | Regeneration within sediments |  |
|  | Nitrogen |  | Surface water | Input of N-enriched groundwater | 17.0 ~ 35.6 μmol L-1 | Stratification and vertical mixing |  |
|  |  |  | Groundwater | Upstream input | 14.9 ~ 54.5 μmol L-1 | Regeneration within sediments |  |
|  |  | Laizhou Bay, China | Surface water | Manure/septic waste | 2.3 mg L-1 | Salinity/ Microbial activities | [11] |
|  |  |  | Groundwater | Ammonium fertilizer/ Soil N | 6.12 mg L-1 | Diverse nitrogen sources/ Slow groundwater renewal/ Surface water recharging |  |
| Non-metallic matter | Fluoride | Southern coastal region of West Bengal, India | Groundwater | Alluvial deposits/ Shallow aquifers/ Anthropogenic activities | 1.29 mg L-1 | Fluorite/gypsum/biotite-bearing minerals | [12] |
| Heavy metals | As |  | Groundwater |  | 0.083 μg L-1 | Leaching/infiltration of arsenic-bearing minerals |  |
|  | As | Tidal channel in Wilmington, Delaware, USA | Sediment | Cumulative of transient hydrologic/geochemical | 5244 ~ 13296 mg kg-1 | Fe and As immobilization | [13] |
|  |  |  | Groundwater | Chemical production facility/ Storm water drainage | 2.45 μg L-1 (1.75 m) | Redox responses/ Water table fluctuations |  |
|  |  |  | Groundwater |  | 25118.4 μg L-1 (3.07 m) | Oxygen deficit/ Release from reductive Fe |  |
|  |  |  | Surface water |  | 44.3 μg L-1 | Water flow/ Aquifer recharge |  |
|  | THg (Total Hg) | China’s coastal wetlands | Soils | Non-ferrous metal smelting /Atmospheric deposition | 96.3 ± 59.3 ng g-1 | Strong affinity of TOC for Hg | [14] |
|  | MeHg (methylmercury) |  | Soils | Non-ferrous metal smelting | 0.12 ± 0.12 ng g-1 | Sulfur cycle/ Sulfate-reducing bacteria/ pH/ Salinity |  |
| Emerging organic matter | Micro- plastics | Punnakayal estuary, India | Surface water | Industrial/tourist sources | 7.8 particles L-1 | Particle density/ Temperature/ Turbulence from tide or waves | [15] |
|  |  |  | Groundwater | Dumped debris from the surface /Leakage from drainage | 4.2 particles L-1 | Aquifer fracture/ Filtration in aquifer lithologies |  |
|  |  | Wen-Rui Tang River watershed, China | Sediment | Discharge of large volumes of untreated domestic/industrial wastewater | 32947 ± 15342 items kg−1 | MPs themselves /River flow/ Buoyancy/ Degradation | [16] |
|  |  | Punnakayal estuary, India | Surface water | Industrial/tourist sources | 7.8 particles L-1 | Particle density/ Temperature/ Turbulence from tide or waves | [15] |
|  | Carbamazepine | Homebush Bay, Australia | Groundwater | Wastewater discharge | 160 ng L-1 | Immobilization | [17] |
|  |  | Hen and Chicken Bay, Australia | Surface water | Wastewater source/ Submarine groundwater discharge | 81 ng L-1 | Water pH/ Biodegradation/ Dilution |  |
|  | Ibuprofen | Rozelle Bay, Australia | Surface water | Urban/ Industrial sources | 150 ng L-1 | Submarine groundwater discharge |  |
|  |  |  | Groundwater | Dissolution/ Desorption | 80 ng L-1 | Biodegradation/ Salinity |  |
|  | PFHxA | Ganges River basin, India | Surface water | Atmospheric inputs | 0.4 ~ 4.7 ng L-1 | Huge catchment /High dilution | [18] |
|  |  |  | Groundwater | Leaching/ Runoff from unpaved surfaces | 0.8 ~ 4.9 ng L-1 | Local sources /River-contaminated levels |  |
|  | PFASs | Gironde estuary, France | Intertidal mudflats | Upstream sources | 2.3 ± 0.7 ng g-1 | Organic carbon fraction/ Sediment grain size/ Iron oxides | [19] |
|  |  |  | Subsurface water | Dissolved from sediment | 0.26 ~ 9.3 ng L-1 | Sediment adsorption/ Suspended particulate matter aggregation/ Salting-out effect |  |
|  |  |  | Bottom water |  | 0.11 ~ 14 ng L-1 | Shifts in PFAS production toward shorter-chain congeners |  |

References

[1] Liu J, Xu G, Zhao S, He J. 2024. Plastisphere microbiomes respiring persistent organohalide pollutants. ‌Environmental Science & Technology 58: 14740-14752. https://doi.org/10.1021/acs.est.4c02251

[2] Tham TT, Anh HQ, Phuong BT, Trinh LT, Thuy NTT, et al. 2020. Contamination status and temporal trends of persistent toxic substances in sediment cores from coastal areas of central Vietnam. Marine Pollution Bulletin 156: 111222. https://doi.org/10.1016/j.marpolbul.2020.111222

[3] Son J, Khuman SN, Park MK, Lee HY, Kim CS, et al. 2024. Distributions of PCDD/Fs, PCBs, and PCNs in coastal sediments collected from major industrial bays in South Korea. Marine Pollution Bulletin 200: 116160. https://doi.org/10.1016/j.marpolbul.2024.116160

[4] Pan R, Zhang N, Li J, Xiao X, Wu H, et al. 2025. Vertical migration characteristics of polychlorinated biphenyls (PCBs) in estuarine and coastal zones under complex environmental conditions. Marine Pollution Bulletin 212: 117520. https://doi.org/10.1016/j.marpolbul.2024.117520

[5] Wu H, Li Q, Wang Y, Hu S. 2023. Distribution, sources, and ecological risk assessment of polychlorinated biphenyls (PCBs) in the estuary of Dagu River, China. Marine Pollution Bulletin 194: 115340. https://doi.org/10.1016/j.marpolbul.2023.115340

[6] Chen C, Lin T, Sun X, Wu Z, Tang J. 2023. Spatiotemporal distribution and particle–water partitioning of polycyclic aromatic hydrocarbons in Bohai Sea, China. Water Research 244: 120440. https://doi.org/10.1016/j.watres.2023.120440

[7] Jiang L, Ma X, Wang Y, Gao W, Liao C, et al. 2022. Land–ocean exchange mechanism of chlorinated paraffins and polycyclic aromatic hydrocarbons with diverse sources in a coastal zone boundary area, North China: The role of regional atmospheric transmission. ‌Environmental Science & Technology 56: 12852-12862. https://doi.org/10.1021/acs.est.2c00742

[8] Iriarte J, Dachs J, Casas G, Martínez-Varela A, Berrojalbiz N, et al. 2023. Snow-dependent biogeochemical cycling of polycyclic aromatic hydrocarbons at coastal Antarctica. ‌Environmental Science & Technology 57: 1625-1636. https://doi.org/10.1021/acs.est.2c05583

[9] Duker RQ, Asare NK, Obodai EA, Adjei JK, Acheampong E, et al. 2024. Ecotoxicological and health risks associated with sediment-bound polycyclic aromatic hydrocarbons in peri-urban closed and open coastal lagoons. Marine Pollution Bulletin 202: 116351. https://doi.org/10.1016/j.marpolbul.2024.116351

[10] Nakajima T, Sugimoto R. 2022. Groundwater-surface water exchange affects nitrogen and phosphorus exports from tideless rivers to a ria coast in the sea of Japan. Journal of Hydrology 612: 128045. https://doi.org/10.1016/j.jhydrol.2022.128045

[11] Xiong G, Zhu X, Liu M, Yang Y, Chen G, et al. 2023. Nitrogen cycle pattern variations during seawater-groundwater-river interactions enhance the nitrogen availability in the coastal earth critical zone. Journal of Hydrology 624: 129932. https://doi.org/10.1016/j.jhydrol.2023.129932

[12] Biswas T, Chandra Pal S, Saha A, Ruidas D. 2023. Arsenic and fluoride exposure in drinking water caused human health risk in coastal groundwater aquifers. Environmental Research 238: 117257. https://doi.org/10.1016/j.envres.2023.117257

[13] Yu X, LeMonte JJ, Li J, Stuckey JW, Sparks DL, et al. 2023. Hydrologic control on arsenic cycling at the groundwater–surface water interface of a tidal channel. ‌Environmental Science & Technology 57: 222-230. https://doi.org/10.1021/acs.est.2c05930

[14] Li Z, Zhou C, Wang Y, He D, Liu M, et al. 2024. Total mercury, methylmercury, and their possible controlling factors in soils of typical coastal wetlands in China. Journal of Hazardous Materials 473: 134711. https://doi.org/10.1016/j.jhazmat.2024.134711

[15] Selvam S, Jesuraja K, Venkatramanan S, Roy PD, Jeyanthi Kumari V. 2021. Hazardous microplastic characteristics and its role as a vector of heavy metal in groundwater and surface water of coastal south India. Journal of Hazardous Materials 402: 123786. https://doi.org/10.1016/j.jhazmat.2020.123786

[16] Wang Z, Su B, Xu X, Di D, Huang H, et al. 2018. Preferential accumulation of small (<300 μm) microplastics in the sediments of a coastal plain river network in eastern China. Water Research 144: 393-401. https://doi.org/10.1016/j.watres.2018.07.050

[17] McKenzie T, Holloway C, Dulai H, Tucker JP, Sugimoto R, et al. 2020. Submarine groundwater discharge: A previously undocumented source of contaminants of emerging concern to the coastal ocean (Sydney, Australia). Marine Pollution Bulletin 160: 111519. https://doi.org/10.1016/j.marpolbul.2020.111519

[18] Sharma BM, Bharat GK, Tayal S, Larssen T, Bečanová J, et al. 2016. Perfluoroalkyl substances (PFAS) in river and ground/drinking water of the Ganges River basin: Emissions and implications for human exposure. Environmental Pollution 208: 704-713. https://doi.org/10.1016/j.envpol.2015.10.050

[19] Munoz G, Budzinski H, Labadie P. 2017. Influence of environmental factors on the fate of legacy and emerging per- and polyfluoroalkyl substances along the salinity/turbidity gradient of a macrotidal estuary. ‌Environmental Science & Technology 51: 12347-12357. https://doi.org/10.1021/acs.est.7b03626